TRACKING AUDIT REPORT

Enhanced Diagnostic Analysis

Simulation: /Users/noasapir/final proj/anomaly_model/anomaly_simulations/region_003/region_003_anomaly_speeding.sim_export_20260517_202258_tracking_audit

Generated: 2026-05-17  |  4 Tracks  |  3 Sensor Types

# 1. Executive Summary

Overall tracking quality is **medium** — RMSE 0.36 m with 42% prediction-only rows; the filter coasts noticeably but stays near truth. The biggest error source is *GPS measurement noise / bias*. Most suspicious behavior: recurring 'sensor_dropout' anomalies (11 occurrences). Best-performing sensor: Camera; weakest: GPS. Weakest time window: t = 16.0–17.0 s (mean err 0.44 m). The system relied heavily on prediction-only Kalman rows (42% of all rows). GPS measurements appear unreliable — see the sensor-quality section.

| Rank | Finding | Impact | Status |
| --- | --- | --- | --- |
| #1 | Overall quality: **medium** — RMSE 0.36 m with 42% prediction-only rows; the filter coasts noticeably but stays near truth | Low | ✓ OK |
| #2 | Biggest error source: GPS measurement noise / bias | Medium | ⚠ Warning |
| #3 | Most suspicious behavior: recurring 'sensor_dropout' anomalies (11 occurrences) | Medium | ⚠ Warning |
| #4 | Best sensor: Camera | Low | ✓ OK |
| #5 | Weakest sensor: GPS | Low | ✓ OK |
| #6 | Weakest time window: t = 16.0–17.0 s (mean err 0.44 m) | Low | ✓ OK |
| #7 | Relied on prediction-only rows: yes (42% of all rows) | Low | ✓ OK |
| #8 | Too few sensor measurements: no | Low | ✓ OK |
| #9 | Suspicious noisy sensor: GPS | Medium | ⚠ Warning |
| #10 | Vehicle fragmentation suspected: no | Low | ✓ OK |

# 2. Sensor Coverage Analysis

Across 4 track(s) the audit recorded 61 GPS, 343 camera and 1976 DAS measurements plus 1670 prediction-only rows (42.3% of all rows). 23 time window(s) had only one sensor type active (see fragmentation / FOV sections for the likely cause).

| Track | GPS | Cam | DAS | Pred | Rows | %pred | Longest meas-gap (s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T000001 | 14 | 68 | 444 | 373 | 887 | 42.1 | 0.04 |
| T000002 | 31 | 159 | 1040 | 893 | 2079 | 43.0 | 0.04 |
| T000003 | 10 | 77 | 298 | 235 | 595 | 39.5 | 0.04 |
| T000004 | 6 | 39 | 194 | 169 | 387 | 43.7 | 0.04 |

23 time window(s) had only one sensor type active (see fragmentation / FOV sections for the likely cause).

ℹ Sensor coverage looks sufficient What happened: GPS=14, Cam=68, DAS=444, prediction-only=42.1%, longest gap=0.04 s. What it means: Track T000001 received a balanced mix of measurements with no large gap. The Kalman has enough evidence to stay close to truth. Why it matters: Healthy coverage is the baseline for trustworthy tracking.

Evidence:

GPS = 14 measurements, Camera = 68, DAS = 444.

Prediction-only rows = 42.1% of the track.

Longest gap between real measurements = 0.04 s (track duration = 17.72 s).

ℹ Sensor coverage looks sufficient What happened: GPS=31, Cam=159, DAS=1040, prediction-only=43.0%, longest gap=0.04 s. What it means: Track T000002 received a balanced mix of measurements with no large gap. The Kalman has enough evidence to stay close to truth. Why it matters: Healthy coverage is the baseline for trustworthy tracking.

Evidence:

GPS = 31 measurements, Camera = 159, DAS = 1040.

Prediction-only rows = 43.0% of the track.

Longest gap between real measurements = 0.04 s (track duration = 41.56 s).

ℹ Sensor coverage looks sufficient What happened: GPS=10, Cam=77, DAS=298, prediction-only=39.5%, longest gap=0.04 s. What it means: Track T000003 received a balanced mix of measurements with no large gap. The Kalman has enough evidence to stay close to truth. Why it matters: Healthy coverage is the baseline for trustworthy tracking.

Evidence:

GPS = 10 measurements, Camera = 77, DAS = 298.

Prediction-only rows = 39.5% of the track.

Longest gap between real measurements = 0.04 s (track duration = 11.88 s).

ℹ Sensor coverage looks sufficient What happened: GPS=6, Cam=39, DAS=194, prediction-only=43.7%, longest gap=0.04 s. What it means: Track T000004 received a balanced mix of measurements with no large gap. The Kalman has enough evidence to stay close to truth. Why it matters: Healthy coverage is the baseline for trustworthy tracking.

Evidence:

GPS = 6 measurements, Camera = 39, DAS = 194.

Prediction-only rows = 43.7% of the track.

Longest gap between real measurements = 0.04 s (track duration = 7.72 s).

# 3. Sensor Quality / Noise Analysis

Per-sensor noise / bias is computed by comparing each accepted measurement against the oracle ground truth at the same timestep. ‘σ vs err’ flags whether the reported sigma matches reality — a ratio far from 1.0 means the sensor model is mis-calibrated.

| Sensor | N | Mean err (m) | Max err (m) | RMSE (m) | Bias x (m) | Bias y (m) | Mean sigma | Sigma vs err |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | 61 | 6.15 | 15.07 | 6.89 | 0.73 | 0.03 | 4.51 | σ ≈ actual |
| Camera | 343 | 0.23 | 0.85 | 0.26 | 0.00 | 0.01 | 0.18 | σ ≈ actual |
| DAS | 1976 | 0.60 | 2.15 | 0.68 | -0.01 | 0.01 | 0.42 | σ ≈ actual |

ℹ GPS behaves as expected What happened: GPS: mean err 6.15, σ 4.51, bias (0.73, 0.03). What it means: GPS mean error and bias are consistent with the reported σ — no immediate noise / calibration concern. Why it matters: This sensor is likely a reliable contributor to the fusion.

Evidence:

61 GPS measurements compared to ground truth.

Mean error = 6.15, max error = 15.07, RMSE = 6.89.

Bias = (Δx=0.73, Δy=0.03).

Mean reported σ = 4.51.

ℹ Camera behaves as expected What happened: Camera: mean err 0.23, σ 0.18, bias (0.00, 0.01). What it means: Camera mean error and bias are consistent with the reported σ — no immediate noise / calibration concern. Why it matters: This sensor is likely a reliable contributor to the fusion.

Evidence:

343 Camera measurements compared to ground truth.

Mean error = 0.23, max error = 0.85, RMSE = 0.26.

Bias = (Δx=0.00, Δy=0.01).

Mean reported σ = 0.18.

ℹ DAS behaves as expected What happened: DAS: mean err 0.60, σ 0.42, bias (-0.01, 0.01). What it means: DAS mean error and bias are consistent with the reported σ — no immediate noise / calibration concern. Why it matters: This sensor is likely a reliable contributor to the fusion.

Evidence:

1976 DAS measurements compared to ground truth.

Mean error = 0.60, max error = 2.15, RMSE = 0.68.

Bias = (Δx=-0.01, Δy=0.01).

Mean reported σ = 0.42.

# 4. Error Spike Investigation

Every detected spike or jump is paired with the local sensor context (what was available before / during / after) and a rule-based explanation that distinguishes confirmed facts from likely causes.

Investigated 1 spike / RMSE / coasting event(s) — each carries an evidence-based explanation below.

⚠ WARNING — Segment-transition coverage gap What happened: Error grew around a segment transition (seg754 → seg1540) while 40% of rows in the window were prediction-only. What it means: Sensor coverage in the new segment is weaker (or starts later) than in the old one, so the filter coasts across the transition. Why it matters: Transitions are sensitive moments — if the next sensor does not match the prediction, the post-transition correction is sharp.

Evidence:

Window: t = 2.36–3.36 s (duration 1.00 s).

Before sensors: Camera, DAS, GPS; during: Camera, DAS, GPS; after: Camera, DAS, GPS.

Mean error before / during / after = 0.19 / 0.35 / 0.23.

Prediction-only rows during the window: 20/50 (40%).

Peak error inside window: 1.00 m.

Segment transition nearby: seg754 → seg1540 at t=2.36 s.

Local DAS SNR samples: mean=18.9, min=11.1.

Possible causes:

Camera / DAS layout has a blind spot at the segment boundary.

Track association may have dropped briefly across the transition.

Next checks:

Plot per-sensor activity on top of the segment-transition vline.

Inspect the audit CSV for skipped measurements right after the transition.

# 5. Track Fragmentation Analysis

4 oracle vehicle(s) were observed across 4 track ID(s).  Each oracle vehicle maps to a single track — no fragmentation suspected.

| Vehicle | # tracks | Tracks | Filtered tracks |
| --- | --- | --- | --- |
| veh001 | 1 | T000001 [0.02-17.74s, 887 rows] | — |
| veh002 | 1 | T000002 [0.02-41.58s, 2079 rows] | — |
| veh003 | 1 | T000003 [0.02-11.90s, 595 rows] | — |
| veh004 | 1 | T000004 [0.02-7.74s, 387 rows] | — |

# 6. Segment Transition Analysis

Compares the segment transitions the report would emit against the transitions implied by oracle lane_id over time.  Mismatches usually indicate a coverage / drift issue rather than a transition-logic bug.

| Track | Vehicle | Reported transitions | Oracle transitions | Match |
| --- | --- | --- | --- | --- |
| T000001 | veh001 | seg1603→seg1545@5.04s; seg1545→seg2500@10.00s; seg2500→seg1538@10.94s; seg1538→seg1540@11.76s; seg1540→seg1990@14.12s | seg1603 → seg1545 → seg2500 → seg1538 → seg1540 → seg1990 | yes |
| T000002 | veh002 | seg71→seg754@18.94s; seg754→seg1540@26.04s; seg1540→seg1990@31.86s | seg71 → seg754 → seg1540 → seg1990 | yes |
| T000003 | veh003 | seg754→seg1540@2.36s; seg1540→seg1990@6.28s | seg754 → seg1540 → seg1990 | yes |
| T000004 | veh004 | seg1540→seg1990@1.86s | seg1540 → seg1990 | yes |

ℹ Segment transition logic appears consistent What happened: Reported transitions match the oracle for every meaningful track. What it means: The segment-assignment pipeline produced the same story as the ground truth. Why it matters: Trustworthy transitions are necessary for any per-segment analysis.

Evidence:

Per-track row in the table above shows match=yes for every row.

# 7. Camera Geometry / FOV Analysis

Per-track camera measurement counts, time windows of camera presence, and a sanity check on the camera σ vs actual error.  Conclusions about FOV are heuristic — the audit data does not carry the camera polygon itself.

| Track | Cam meas | Cam window (s) | Pre-tr / post-tr | Mean σ | Mean err |
| --- | --- | --- | --- | --- | --- |
| T000001 | 68 | 12.48 | 0/68 | 0.17 | 0.23 |
| T000002 | 159 | 26.92 | 10/149 | 0.19 | 0.23 |
| T000003 | 77 | 11.26 | 19/58 | 0.18 | 0.21 |
| T000004 | 39 | 6.58 | 14/25 | 0.19 | 0.21 |

# 8. DAS Quality Analysis

DAS measurement counts, SNR distribution, and the Kalman error in DAS-only windows.  Where SNR data is available, correlations with tracking error are flagged as evidence of σ-from-SNR mis-calibration.

| Track | DAS meas | Mean SNR | Min SNR | Low-SNR rows | Mean err |
| --- | --- | --- | --- | --- | --- |
| T000001 | 444 | 35.45 | 8.87 | 0 | 0.33 |
| T000002 | 1040 | 29.71 | 7.41 | 0 | 0.31 |
| T000003 | 298 | 31.42 | 11.11 | 0 | 0.26 |
| T000004 | 194 | 70.55 | 16.15 | 0 | 0.27 |

# 9. Kalman Behavior Analysis

For each track the Kalman is evaluated on: how often it received real updates (vs prediction), whether σ grows in gaps and shrinks after measurements, and whether the reported σ matches the actual error.  Strong mismatches indicate a tuning issue — not a bug.

| Track | % pred-only | σ growth in gaps | σ shrink after meas | σ vs actual err |
| --- | --- | --- | --- | --- |
| T000001 | 42.1 | 203/373 | 249/373 | σ ≈ err (healthy) |
| T000002 | 43.0 | 467/893 | 628/893 | σ ≈ err (healthy) |
| T000003 | 39.5 | 68/235 | 124/235 | σ ≈ err (healthy) |
| T000004 | 43.7 | 91/169 | 120/169 | σ ≈ err (healthy) |

ℹ Kalman behaves correctly What happened: σ tracks actual error, σ grows during gaps, shrinks after measurements. What it means: All meaningful tracks show the textbook Kalman pattern: uncertainty rises while coasting and falls when sensor data returns, and the reported uncertainty matches the actual error well. Why it matters: No tuning intervention needed for the filter itself.

Evidence:

4/4 tracks classified healthy.

# 10. Recommendations & Next Investigations

Concrete checks aggregated from the diagnostic sections.  Items are ordered by severity (critical → warning → info) and de-duplicated.  Each is linked to the interpretation that suggested it.

(Segment-transition coverage gap) Plot per-sensor activity on top of the segment-transition vline.

(Segment-transition coverage gap) Inspect the audit CSV for skipped measurements right after the transition.

# 11. Track Overview

4 track(s) recorded.  Global RMSE: 0.36 m.  Global max error: 1.25 m.  Prediction-only rows: 1670.

| Track | Vehicle | Duration (s) | RMSE (m) | Max err (m) | GPS | Camera | DAS | Pred-only | % Pred | Segments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T000001 | veh001 | 17.72 | 0.37 | 1.06 | 14 | 68 | 444 | 373 | 42% | seg1603 → seg1545 → seg2500 → seg1538 → seg1540 → seg1990 |
| T000002 | veh002 | 41.56 | 0.35 | 1.02 | 31 | 159 | 1040 | 893 | 42% | seg71 → seg754 → seg1540 → seg1990 |
| T000003 | veh003 | 11.88 | 0.30 | 1.00 | 10 | 77 | 298 | 235 | 39% | seg754 → seg1540 → seg1990 |
| T000004 | veh004 | 7.72 | 0.29 | 0.73 | 6 | 39 | 194 | 169 | 43% | seg1540 → seg1990 |

## Segment Transitions

11 transition(s) across all tracks.

| Track | Time (s) | From segment | To segment |
| --- | --- | --- | --- |
| T000001 | 5.04 | seg1603 | seg1545 |
| T000001 | 10.00 | seg1545 | seg2500 |
| T000001 | 10.94 | seg2500 | seg1538 |
| T000001 | 11.76 | seg1538 | seg1540 |
| T000001 | 14.12 | seg1540 | seg1990 |
| T000002 | 18.94 | seg71 | seg754 |
| T000002 | 26.04 | seg754 | seg1540 |
| T000002 | 31.86 | seg1540 | seg1990 |
| T000003 | 2.36 | seg754 | seg1540 |
| T000003 | 6.28 | seg1540 | seg1990 |
| T000004 | 1.86 | seg1540 | seg1990 |

# 12. Ghost-Track Filter Analysis

The simulator's tracker assigns a new tentative track to every fiber-optic DAS hit that falls outside the association gate of all existing tracks.  In dense sensor deployments this produces large numbers of single-hit "ghost tracks" — tracks that were never observed a second time and therefore carry no useful trajectory information.  Before export, a ghost-track filter suppresses any track that received fewer than 5 tracker updates (the MIN_TRACK_UPDATES threshold).  The table and breakdown below document every filtered track so the analyst can verify that no real vehicle was incorrectly discarded.

⚠ WARNING — No track events No track.update events were found in the event log. Ghost-track statistics cannot be computed for this run.

# 13. Sensor σ Calibration Check

Compares each sensor's declared measurement noise (sigma_m) against actual error from ground truth.  A well-calibrated sensor has mean actual error ≈ mean declared σ.  If actual error > 1.2×σ the filter over-trusts that sensor and the R-matrix entry should be increased.

| Sensor | N meas. | Mean actual err (m) | Max actual err (m) | RMSE (m) | Mean declared σ (m) | Actual / σ ratio | % over-trusted | Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | 87 | 6.172 | 15.068 | 6.888 | 4.512 | 1.37 | 56.3% | ⚠ Over-trusted |
| Camera | 556 | 0.230 | 0.904 | 0.269 | 0.181 | 1.27 | 47.8% | ⚠ Over-trusted |
| DAS | 1976 | 0.596 | 2.149 | 0.681 | 0.419 | 1.42 | 58.8% | ⚠ Over-trusted |

⚠ CALIBRATION WARNING GPS: actual error is 1.37× declared σ (mean err 6.17 m vs σ 4.51 m). Increase R_gps in the Kalman noise matrix.

⚠ CALIBRATION WARNING Camera: actual error is 1.27× declared σ (mean err 0.23 m vs σ 0.18 m). Increase R_camera in the Kalman noise matrix.

⚠ CALIBRATION WARNING DAS: actual error is 1.42× declared σ (mean err 0.60 m vs σ 0.42 m). Increase R_das in the Kalman noise matrix.

# 14. Per-Axis Error Breakdown (X vs Y)

Scalar RMSE hides directional bias.  A dominant Y error (lateral) usually points to insufficient lateral process noise (Q_yy too small) or missing sensors that constrain cross-track position.  A dominant X error suggests speed-model mismatch.

## Track T000001 — veh001

| Axis | Mean ¦error¦ (m) | RMSE (m) | Signed bias (m) | Dominant? |
| --- | --- | --- | --- | --- |
| X (longitudinal) | 0.243 | 0.305 | +0.010 | Yes |
| Y (lateral) | 0.183 | 0.263 | +0.017 | — |

✓ X and Y errors are balanced — no strong axis bias.

## Track T000002 — veh002

| Axis | Mean ¦error¦ (m) | RMSE (m) | Signed bias (m) | Dominant? |
| --- | --- | --- | --- | --- |
| X (longitudinal) | 0.215 | 0.279 | -0.018 | Yes |
| Y (lateral) | 0.183 | 0.240 | -0.011 | — |

✓ X and Y errors are balanced — no strong axis bias.

## Track T000003 — veh003

| Axis | Mean ¦error¦ (m) | RMSE (m) | Signed bias (m) | Dominant? |
| --- | --- | --- | --- | --- |
| X (longitudinal) | 0.190 | 0.253 | -0.000 | Yes |
| Y (lateral) | 0.143 | 0.189 | +0.038 | — |

✓ X and Y errors are balanced — no strong axis bias.

## Track T000004 — veh004

| Axis | Mean ¦error¦ (m) | RMSE (m) | Signed bias (m) | Dominant? |
| --- | --- | --- | --- | --- |
| X (longitudinal) | 0.211 | 0.265 | -0.019 | Yes |
| Y (lateral) | 0.131 | 0.181 | -0.015 | — |

⚠ X (longitudinal) error dominates by 1.6×. Consider increasing Q_xx (longitudinal process noise) or reviewing the vehicle speed model.

# 15. Sensor Coverage Timeline

Each row = one second of simulation.  ✓ = at least one accepted measurement;  — = sensor silent (filter coasted).  Seconds where ALL sensors are silent are the primary source of position drift.

## Track T000001 — veh001

| Second (s) | GPS | Camera | DAS | Status |
| --- | --- | --- | --- | --- |
| 0–1 | ✓ | — | ✓ | ✓ sensor active |
| 1–2 | ✓ | — | ✓ | ✓ sensor active |
| 2–3 | ✓ | — | ✓ | ✓ sensor active |
| 3–4 | — | — | ✓ | ✓ sensor active |
| 4–5 | ✓ | — | ✓ | ✓ sensor active |
| 5–6 | ✓ | ✓ | ✓ | ✓ sensor active |
| 6–7 | ✓ | ✓ | ✓ | ✓ sensor active |
| 7–8 | ✓ | ✓ | ✓ | ✓ sensor active |
| 8–9 | ✓ | — | ✓ | ✓ sensor active |
| 9–10 | — | ✓ | ✓ | ✓ sensor active |
| 10–11 | ✓ | ✓ | ✓ | ✓ sensor active |
| 11–12 | ✓ | ✓ | ✓ | ✓ sensor active |
| 12–13 | ✓ | ✓ | ✓ | ✓ sensor active |
| 13–14 | ✓ | ✓ | ✓ | ✓ sensor active |
| 14–15 | — | ✓ | ✓ | ✓ sensor active |
| 15–16 | ✓ | ✓ | ✓ | ✓ sensor active |
| 16–17 | ✓ | ✓ | ✓ | ✓ sensor active |
| 17–18 | — | ✓ | ✓ | ✓ sensor active |
| 18–19 | — | — | — | ✗ prediction only |

Dead seconds (all sensors silent): 1 / 19 (5%)

## Track T000002 — veh002

| Second (s) | GPS | Camera | DAS | Status |
| --- | --- | --- | --- | --- |
| 0–1 | ✓ | — | ✓ | ✓ sensor active |
| 1–2 | ✓ | — | ✓ | ✓ sensor active |
| 2–3 | ✓ | — | ✓ | ✓ sensor active |
| 3–4 | ✓ | — | ✓ | ✓ sensor active |
| 4–5 | ✓ | — | ✓ | ✓ sensor active |
| 5–6 | ✓ | — | ✓ | ✓ sensor active |
| 6–7 | — | — | ✓ | ✓ sensor active |
| 7–8 | — | — | ✓ | ✓ sensor active |
| 8–9 | — | — | ✓ | ✓ sensor active |
| 9–10 | — | — | ✓ | ✓ sensor active |
| 10–11 | ✓ | — | ✓ | ✓ sensor active |
| 11–12 | ✓ | — | ✓ | ✓ sensor active |
| 12–13 | ✓ | — | ✓ | ✓ sensor active |
| 13–14 | ✓ | ✓ | ✓ | ✓ sensor active |
| 14–15 | ✓ | ✓ | ✓ | ✓ sensor active |
| 15–16 | ✓ | — | ✓ | ✓ sensor active |
| 16–17 | — | ✓ | ✓ | ✓ sensor active |
| 17–18 | — | ✓ | ✓ | ✓ sensor active |
| 18–19 | — | ✓ | ✓ | ✓ sensor active |
| 19–20 | — | ✓ | ✓ | ✓ sensor active |
| 20–21 | ✓ | ✓ | ✓ | ✓ sensor active |
| 21–22 | ✓ | ✓ | ✓ | ✓ sensor active |
| 22–23 | ✓ | ✓ | ✓ | ✓ sensor active |
| 23–24 | ✓ | ✓ | ✓ | ✓ sensor active |
| 24–25 | ✓ | ✓ | ✓ | ✓ sensor active |
| 25–26 | ✓ | ✓ | ✓ | ✓ sensor active |
| 26–27 | ✓ | ✓ | ✓ | ✓ sensor active |
| 27–28 | ✓ | ✓ | ✓ | ✓ sensor active |
| 28–29 | ✓ | ✓ | ✓ | ✓ sensor active |
| 29–30 | ✓ | ✓ | ✓ | ✓ sensor active |
| 30–31 | ✓ | ✓ | ✓ | ✓ sensor active |
| 31–32 | ✓ | ✓ | ✓ | ✓ sensor active |
| 32–33 | — | ✓ | ✓ | ✓ sensor active |
| 33–34 | ✓ | ✓ | ✓ | ✓ sensor active |
| 34–35 | ✓ | ✓ | ✓ | ✓ sensor active |
| 35–36 | ✓ | ✓ | ✓ | ✓ sensor active |
| 36–37 | ✓ | ✓ | ✓ | ✓ sensor active |
| 37–38 | ✓ | ✓ | ✓ | ✓ sensor active |
| 38–39 | ✓ | ✓ | ✓ | ✓ sensor active |
| 39–40 | ✓ | ✓ | ✓ | ✓ sensor active |
| 40–41 | — | ✓ | ✓ | ✓ sensor active |
| 41–42 | — | — | ✓ | ✓ sensor active |
| 42–43 | — | — | — | ✗ prediction only |

Dead seconds (all sensors silent): 1 / 43 (2%)

## Track T000003 — veh003

| Second (s) | GPS | Camera | DAS | Status |
| --- | --- | --- | --- | --- |
| 0–1 | ✓ | ✓ | ✓ | ✓ sensor active |
| 1–2 | ✓ | ✓ | ✓ | ✓ sensor active |
| 2–3 | ✓ | ✓ | ✓ | ✓ sensor active |
| 3–4 | ✓ | ✓ | ✓ | ✓ sensor active |
| 4–5 | ✓ | ✓ | ✓ | ✓ sensor active |
| 5–6 | ✓ | ✓ | ✓ | ✓ sensor active |
| 6–7 | ✓ | ✓ | ✓ | ✓ sensor active |
| 7–8 | — | ✓ | ✓ | ✓ sensor active |
| 8–9 | ✓ | ✓ | ✓ | ✓ sensor active |
| 9–10 | ✓ | ✓ | ✓ | ✓ sensor active |
| 10–11 | ✓ | ✓ | ✓ | ✓ sensor active |
| 11–12 | — | ✓ | ✓ | ✓ sensor active |
| 12–13 | — | — | — | ✗ prediction only |

Dead seconds (all sensors silent): 1 / 13 (7%)

## Track T000004 — veh004

| Second (s) | GPS | Camera | DAS | Status |
| --- | --- | --- | --- | --- |
| 0–1 | ✓ | ✓ | ✓ | ✓ sensor active |
| 1–2 | ✓ | ✓ | ✓ | ✓ sensor active |
| 2–3 | — | ✓ | ✓ | ✓ sensor active |
| 3–4 | ✓ | ✓ | ✓ | ✓ sensor active |
| 4–5 | ✓ | ✓ | ✓ | ✓ sensor active |
| 5–6 | ✓ | ✓ | ✓ | ✓ sensor active |
| 6–7 | ✓ | ✓ | ✓ | ✓ sensor active |
| 7–8 | — | — | ✓ | ✓ sensor active |
| 8–9 | — | — | — | ✗ prediction only |

Dead seconds (all sensors silent): 1 / 9 (11%)

# 16. Cold-Start Latency

Time from the first trajectory row to the first moment the position error drops below 0.5 m.  A long cold-start indicates the filter is initialized far from the true position or the initial covariance P₀ is too small.

| Track | Vehicle | Track start (s) | First error < 0.5 m at (s) | Cold-start latency (s) | Initial error (m) |
| --- | --- | --- | --- | --- | --- |
| T000001 | veh001 | 0.020 | 0.020 | 0.000 s | 0.213 |
| T000002 | veh002 | 0.020 | 0.060 | 0.040 s | 0.805 |
| T000003 | veh003 | 0.020 | 0.020 | 0.000 s | 0.164 |
| T000004 | veh004 | 0.020 | 0.020 | 0.000 s | 0.031 |

# 17. Speed vs Position Error

Position error bucketed by estimated vehicle speed.  If error grows with speed, the process noise Q or motion model is misspecified for high-speed dynamics.  If error is flat, sensor dropout is the dominant driver.

| Speed bucket | N rows | Mean error (m) | Max error (m) | Trend vs prev |
| --- | --- | --- | --- | --- |
| 0–5 m/s | 29 | 0.577 | 1.089 | — |
| 5–10 m/s | 1966 | 0.311 | 1.249 | ↓ shrinking |
| 10–15 m/s | 1051 | 0.281 | 1.206 | → stable |
| 15–20 m/s | 252 | 0.387 | 1.072 | ↑ growing |
| 20+ m/s | 650 | 0.307 | 0.941 | ↓ shrinking |

✓ Error does not strongly correlate with speed — sensor dropout is the primary error driver, not model mismatch.