TRACKING AUDIT REPORT

Enhanced Diagnostic Analysis

Simulation: /Users/noasapir/final proj/anomaly_model/anomaly_simulations/region_120/region_120_anomaly_collision.sim_export_20260517_204402_tracking_audit

Generated: 2026-05-17  |  4 Tracks  |  3 Sensor Types

# 1. Executive Summary

Overall tracking quality is **medium** — RMSE 0.41 m with 48% prediction-only rows; the filter coasts noticeably but stays near truth. The biggest error source is *GPS measurement noise / bias*. Most suspicious behavior: recurring 'sensor_dropout' anomalies (6 occurrences). Best-performing sensor: Camera; weakest: GPS. Weakest time window: t = 60.0–61.0 s (mean err 0.51 m). The system relied heavily on prediction-only Kalman rows (48% of all rows). GPS measurements appear unreliable — see the sensor-quality section.

| Rank | Finding | Impact | Status |
| --- | --- | --- | --- |
| #1 | Overall quality: **medium** — RMSE 0.41 m with 48% prediction-only rows; the filter coasts noticeably but stays near truth | Low | ✓ OK |
| #2 | Biggest error source: GPS measurement noise / bias | Medium | ⚠ Warning |
| #3 | Most suspicious behavior: recurring 'sensor_dropout' anomalies (6 occurrences) | Medium | ⚠ Warning |
| #4 | Best sensor: Camera | Low | ✓ OK |
| #5 | Weakest sensor: GPS | Low | ✓ OK |
| #6 | Weakest time window: t = 60.0–61.0 s (mean err 0.51 m) | Low | ✓ OK |
| #7 | Relied on prediction-only rows: yes (48% of all rows) | Low | ✓ OK |
| #8 | Too few sensor measurements: no | Low | ✓ OK |
| #9 | Suspicious noisy sensor: GPS | Medium | ⚠ Warning |
| #10 | Vehicle fragmentation suspected: no | Low | ✓ OK |

# 2. Sensor Coverage Analysis

Across 4 track(s) the audit recorded 358 GPS, 104 camera and 9317 DAS measurements plus 9012 prediction-only rows (48.4% of all rows). Camera contributes very few measurements while DAS dominates — DAS is the de-facto primary sensor.

| Track | GPS | Cam | DAS | Pred | Rows | %pred | Longest meas-gap (s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T000001 | 117 | 1 | 3000 | 2926 | 5999 | 48.8 | 0.04 |
| T000002 | 119 | 0 | 3000 | 2926 | 5999 | 48.8 | 0.04 |
| T000003 | 111 | 23 | 3000 | 2907 | 5999 | 48.5 | 0.04 |
| T000004 | 11 | 80 | 317 | 253 | 633 | 40.0 | 0.04 |

Camera contributes very few measurements while DAS dominates — DAS is the de-facto primary sensor.

343 time window(s) had only one sensor type active (see fragmentation / FOV sections for the likely cause).

ℹ Sensor coverage looks sufficient What happened: GPS=117, Cam=1, DAS=3000, prediction-only=48.8%, longest gap=0.04 s. What it means: Track T000001 received a balanced mix of measurements with no large gap. The Kalman has enough evidence to stay close to truth. Why it matters: Healthy coverage is the baseline for trustworthy tracking.

Evidence:

GPS = 117 measurements, Camera = 1, DAS = 3000.

Prediction-only rows = 48.8% of the track.

Longest gap between real measurements = 0.04 s (track duration = 119.96 s).

⚠ WARNING — DAS-only track (no camera support) What happened: GPS=119, Cam=0, DAS=3000, prediction-only=48.8%, longest gap=0.04 s. What it means: Track T000002 never received a camera measurement. The Kalman fuses only DAS (and possibly GPS), so spatial precision is bounded by the DAS sigma model. Why it matters: Camera updates are usually the most precise; their absence shifts the burden to DAS and prediction, which typically grows error.

Evidence:

GPS = 119 measurements, Camera = 0, DAS = 3000.

Prediction-only rows = 48.8% of the track.

Longest gap between real measurements = 0.04 s (track duration = 119.96 s).

Possible causes:

Vehicle was outside camera FOV for the whole track lifetime.

Camera measurements were filtered before entering Kalman.

Next checks:

Check camera FOV vs vehicle path for the relevant time window.

ℹ Sensor coverage looks sufficient What happened: GPS=111, Cam=23, DAS=3000, prediction-only=48.5%, longest gap=0.04 s. What it means: Track T000003 received a balanced mix of measurements with no large gap. The Kalman has enough evidence to stay close to truth. Why it matters: Healthy coverage is the baseline for trustworthy tracking.

Evidence:

GPS = 111 measurements, Camera = 23, DAS = 3000.

Prediction-only rows = 48.5% of the track.

Longest gap between real measurements = 0.04 s (track duration = 119.96 s).

ℹ Sensor coverage looks sufficient What happened: GPS=11, Cam=80, DAS=317, prediction-only=40.0%, longest gap=0.04 s. What it means: Track T000004 received a balanced mix of measurements with no large gap. The Kalman has enough evidence to stay close to truth. Why it matters: Healthy coverage is the baseline for trustworthy tracking.

Evidence:

GPS = 11 measurements, Camera = 80, DAS = 317.

Prediction-only rows = 40.0% of the track.

Longest gap between real measurements = 0.04 s (track duration = 12.64 s).

# 3. Sensor Quality / Noise Analysis

Per-sensor noise / bias is computed by comparing each accepted measurement against the oracle ground truth at the same timestep. ‘σ vs err’ flags whether the reported sigma matches reality — a ratio far from 1.0 means the sensor model is mis-calibrated.

| Sensor | N | Mean err (m) | Max err (m) | RMSE (m) | Bias x (m) | Bias y (m) | Mean sigma | Sigma vs err |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | 358 | 5.04 | 15.18 | 5.74 | 0.20 | 0.36 | 4.20 | σ ≈ actual |
| Camera | 104 | 0.23 | 0.71 | 0.27 | 0.01 | -0.00 | 0.19 | σ ≈ actual |
| DAS | 9317 | 0.56 | 2.26 | 0.64 | -0.01 | -0.01 | 0.40 | σ ≈ actual |

ℹ GPS behaves as expected What happened: GPS: mean err 5.04, σ 4.20, bias (0.20, 0.36). What it means: GPS mean error and bias are consistent with the reported σ — no immediate noise / calibration concern. Why it matters: This sensor is likely a reliable contributor to the fusion.

Evidence:

358 GPS measurements compared to ground truth.

Mean error = 5.04, max error = 15.18, RMSE = 5.74.

Bias = (Δx=0.20, Δy=0.36).

Mean reported σ = 4.20.

ℹ Camera behaves as expected What happened: Camera: mean err 0.23, σ 0.19, bias (0.01, -0.00). What it means: Camera mean error and bias are consistent with the reported σ — no immediate noise / calibration concern. Why it matters: This sensor is likely a reliable contributor to the fusion.

Evidence:

104 Camera measurements compared to ground truth.

Mean error = 0.23, max error = 0.71, RMSE = 0.27.

Bias = (Δx=0.01, Δy=-0.00).

Mean reported σ = 0.19.

ℹ DAS behaves as expected What happened: DAS: mean err 0.56, σ 0.40, bias (-0.01, -0.01). What it means: DAS mean error and bias are consistent with the reported σ — no immediate noise / calibration concern. Why it matters: This sensor is likely a reliable contributor to the fusion.

Evidence:

9317 DAS measurements compared to ground truth.

Mean error = 0.56, max error = 2.26, RMSE = 0.64.

Bias = (Δx=-0.01, Δy=-0.01).

Mean reported σ = 0.40.

# 4. Error Spike Investigation

Every detected spike or jump is paired with the local sensor context (what was available before / during / after) and a rule-based explanation that distinguishes confirmed facts from likely causes.

Investigated 3 spike / RMSE / coasting event(s) — each carries an evidence-based explanation below.

ℹ Normal recovery after dropout What happened: Error rose to 0.94 mid-window and recovered to 0.37 once measurements returned. What it means: This is the textbook signature of a temporary sensor coverage loss followed by Kalman re-acquisition. Why it matters: Recovered errors are usually not bugs — they are diagnostics of coverage.

Evidence:

Window: t = -0.48–0.62 s (duration 1.10 s).

Before sensors: none; during: Camera, DAS; after: DAS.

Mean error before / during / after = n/a / 0.94 / 0.37.

Prediction-only rows during the window: 15/30 (50%).

Peak error inside window: 1.85 m.

Local DAS SNR samples: mean=55.2, min=22.9.

Next checks:

Confirm sigma_pos_m drops once measurements return.

⚠ WARNING — Possible camera FOV boundary issue What happened: Error rose from 0.17 to a peak of 2.79 when the camera stopped contributing measurements. What it means: The filter lost its strongest spatial anchor (camera) and started relying on DAS / prediction. The rise in error is the expected consequence of switching to a less-precise sensor mix. Why it matters: If the vehicle keeps moving outside camera coverage, the error will grow until either DAS gets a lock or the camera sees it again.

Evidence:

Window: t = 2.64–2.68 s (duration 0.04 s).

Before sensors: Camera, DAS, GPS; during: DAS; after: Camera, DAS, GPS.

Mean error before / during / after = 0.17 / 2.27 / 0.34.

Prediction-only rows during the window: 2/3 (67%).

Peak error inside window: 2.79 m.

Segment transition nearby: seg2801 → seg2493 at t=2.64 s.

Camera was present before but disappeared inside the window.

Local DAS SNR samples: mean=69.4, min=26.1.

Possible causes:

Vehicle crossed the edge of the camera FOV.

Camera sigma model may be too optimistic close to the edge.

Next checks:

Overlay vehicle position on the camera FOV polygon.

Add or extend a camera covering the post-transition area.

⚠ WARNING — Segment-transition coverage gap What happened: Error grew around a segment transition (seg2801 → seg2493) while 46% of rows in the window were prediction-only. What it means: Sensor coverage in the new segment is weaker (or starts later) than in the old one, so the filter coasts across the transition. Why it matters: Transitions are sensitive moments — if the next sensor does not match the prediction, the post-transition correction is sharp.

Evidence:

Window: t = 2.64–3.64 s (duration 1.00 s).

Before sensors: Camera, DAS, GPS; during: Camera, DAS, GPS; after: Camera, DAS, GPS.

Mean error before / during / after = 0.17 / 0.46 / 0.25.

Prediction-only rows during the window: 23/50 (46%).

Peak error inside window: 2.79 m.

Segment transition nearby: seg2801 → seg2493 at t=2.64 s.

Local DAS SNR samples: mean=121.2, min=26.1.

Possible causes:

Camera / DAS layout has a blind spot at the segment boundary.

Track association may have dropped briefly across the transition.

Next checks:

Plot per-sensor activity on top of the segment-transition vline.

Inspect the audit CSV for skipped measurements right after the transition.

# 5. Track Fragmentation Analysis

4 oracle vehicle(s) were observed across 4 track ID(s).  Each oracle vehicle maps to a single track — no fragmentation suspected.

| Vehicle | # tracks | Tracks | Filtered tracks |
| --- | --- | --- | --- |
| veh001 | 1 | T000001 [0.02-119.98s, 5999 rows] | — |
| veh002 | 1 | T000002 [0.02-119.98s, 5999 rows] | — |
| veh003 | 1 | T000003 [0.02-119.98s, 5999 rows] | — |
| veh004 | 1 | T000004 [0.02-12.66s, 633 rows] | — |

# 6. Segment Transition Analysis

Compares the segment transitions the report would emit against the transitions implied by oracle lane_id over time.  Mismatches usually indicate a coverage / drift issue rather than a transition-logic bug.

| Track | Vehicle | Reported transitions | Oracle transitions | Match |
| --- | --- | --- | --- | --- |
| T000001 | veh001 | — | seg2342 | yes |
| T000002 | veh002 | — | seg2342 | yes |
| T000003 | veh003 | seg2061→seg2342@9.82s | seg2061 → seg2342 | yes |
| T000004 | veh004 | seg2801→seg2493@2.64s | seg2801 → seg2493 | yes |

ℹ Segment transition logic appears consistent What happened: Reported transitions match the oracle for every meaningful track. What it means: The segment-assignment pipeline produced the same story as the ground truth. Why it matters: Trustworthy transitions are necessary for any per-segment analysis.

Evidence:

Per-track row in the table above shows match=yes for every row.

# 7. Camera Geometry / FOV Analysis

Per-track camera measurement counts, time windows of camera presence, and a sanity check on the camera σ vs actual error.  Conclusions about FOV are heuristic — the audit data does not carry the camera polygon itself.

| Track | Cam meas | Cam window (s) | Pre-tr / post-tr | Mean σ | Mean err |
| --- | --- | --- | --- | --- | --- |
| T000001 | 1 | 0.00 | n/a | 0.23 | 0.20 |
| T000002 | 0 | — | — | — | — |
| T000003 | 23 | 5.04 | 0/23 | 0.16 | 0.20 |
| T000004 | 80 | 12.34 | 24/56 | 0.19 | 0.24 |

ℹ No camera coverage on this track What happened: Track T000002 received no camera measurements. What it means: The camera FOV did not include this vehicle at any time during the track. Why it matters: Without camera, the spatial precision of fusion is bounded by DAS / GPS.

Evidence:

Camera count: 0 over track t=[0.02, 119.98] s.

Possible causes:

Vehicle path lies entirely outside the camera FOV.

All camera measurements were filtered before the audit.

Next checks:

Plot camera FOV polygon over the vehicle path.

Inspect the audit CSV for camera-row skip reasons.

# 8. DAS Quality Analysis

DAS measurement counts, SNR distribution, and the Kalman error in DAS-only windows.  Where SNR data is available, correlations with tracking error are flagged as evidence of σ-from-SNR mis-calibration.

| Track | DAS meas | Mean SNR | Min SNR | Low-SNR rows | Mean err |
| --- | --- | --- | --- | --- | --- |
| T000001 | 3000 | 35.71 | 8.65 | 0 | 0.36 |
| T000002 | 3000 | 23.17 | 23.17 | 0 | 0.31 |
| T000003 | 3000 | 37.74 | 10.50 | 0 | 0.35 |
| T000004 | 317 | 164.74 | 26.11 | 0 | 0.24 |

# 9. Kalman Behavior Analysis

For each track the Kalman is evaluated on: how often it received real updates (vs prediction), whether σ grows in gaps and shrinks after measurements, and whether the reported σ matches the actual error.  Strong mismatches indicate a tuning issue — not a bug.

| Track | % pred-only | σ growth in gaps | σ shrink after meas | σ vs actual err |
| --- | --- | --- | --- | --- |
| T000001 | 48.8 | 2669/2926 | 2925/2926 | σ ≈ err (healthy) |
| T000002 | 48.8 | 2926/2926 | 2926/2926 | σ ≈ err (healthy) |
| T000003 | 48.5 | 2650/2907 | 2888/2907 | σ ≈ err (healthy) |
| T000004 | 40.0 | 116/253 | 190/253 | σ ≈ err (healthy) |

ℹ Kalman behaves correctly What happened: σ tracks actual error, σ grows during gaps, shrinks after measurements. What it means: All meaningful tracks show the textbook Kalman pattern: uncertainty rises while coasting and falls when sensor data returns, and the reported uncertainty matches the actual error well. Why it matters: No tuning intervention needed for the filter itself.

Evidence:

4/4 tracks classified healthy.

# 10. Recommendations & Next Investigations

Concrete checks aggregated from the diagnostic sections.  Items are ordered by severity (critical → warning → info) and de-duplicated.  Each is linked to the interpretation that suggested it.

(DAS-only track (no camera support)) Check camera FOV vs vehicle path for the relevant time window.

(Possible camera FOV boundary issue) Overlay vehicle position on the camera FOV polygon.

(Possible camera FOV boundary issue) Add or extend a camera covering the post-transition area.

(Segment-transition coverage gap) Plot per-sensor activity on top of the segment-transition vline.

(Segment-transition coverage gap) Inspect the audit CSV for skipped measurements right after the transition.

(Normal recovery after dropout) Confirm sigma_pos_m drops once measurements return.

(No camera coverage on this track) Plot camera FOV polygon over the vehicle path.

(No camera coverage on this track) Inspect the audit CSV for camera-row skip reasons.

# 11. Track Overview

4 track(s) recorded.  Global RMSE: 0.41 m.  Global max error: 2.79 m.  Prediction-only rows: 9012.

| Track | Vehicle | Duration (s) | RMSE (m) | Max err (m) | GPS | Camera | DAS | Pred-only | % Pred | Segments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T000001 | veh001 | 119.96 | 0.41 | 1.57 | 117 | 1 | 3000 | 2926 | 48% | seg2342 |
| T000002 | veh002 | 119.96 | 0.35 | 1.08 | 119 | 0 | 3000 | 2926 | 48% | seg2342 |
| T000003 | veh003 | 119.96 | 0.40 | 1.22 | 111 | 23 | 3000 | 2907 | 48% | seg2061 → seg2342 |
| T000004 | veh004 | 12.64 | 0.29 | 1.91 | 11 | 80 | 317 | 253 | 39% | seg2801 → seg2493 |

## Segment Transitions

2 transition(s) across all tracks.

| Track | Time (s) | From segment | To segment |
| --- | --- | --- | --- |
| T000003 | 9.82 | seg2061 | seg2342 |
| T000004 | 2.64 | seg2801 | seg2493 |

# 12. Ghost-Track Filter Analysis

The simulator's tracker assigns a new tentative track to every fiber-optic DAS hit that falls outside the association gate of all existing tracks.  In dense sensor deployments this produces large numbers of single-hit "ghost tracks" — tracks that were never observed a second time and therefore carry no useful trajectory information.  Before export, a ghost-track filter suppresses any track that received fewer than 5 tracker updates (the MIN_TRACK_UPDATES threshold).  The table and breakdown below document every filtered track so the analyst can verify that no real vehicle was incorrectly discarded.

⚠ WARNING — No track events No track.update events were found in the event log. Ghost-track statistics cannot be computed for this run.

# 13. Sensor σ Calibration Check

Compares each sensor's declared measurement noise (sigma_m) against actual error from ground truth.  A well-calibrated sensor has mean actual error ≈ mean declared σ.  If actual error > 1.2×σ the filter over-trusts that sensor and the R-matrix entry should be increased.

| Sensor | N meas. | Mean actual err (m) | Max actual err (m) | RMSE (m) | Mean declared σ (m) | Actual / σ ratio | % over-trusted | Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | 373 | 5.068 | 15.184 | 5.761 | 4.198 | 1.21 | 44.0% | ⚠ Over-trusted |
| Camera | 142 | 0.244 | 0.712 | 0.279 | 0.189 | 1.29 | 51.4% | ⚠ Over-trusted |
| DAS | 9317 | 0.562 | 2.259 | 0.643 | 0.402 | 1.40 | 56.1% | ⚠ Over-trusted |

⚠ CALIBRATION WARNING GPS: actual error is 1.21× declared σ (mean err 5.07 m vs σ 4.20 m). Increase R_gps in the Kalman noise matrix.

⚠ CALIBRATION WARNING Camera: actual error is 1.29× declared σ (mean err 0.24 m vs σ 0.19 m). Increase R_camera in the Kalman noise matrix.

⚠ CALIBRATION WARNING DAS: actual error is 1.40× declared σ (mean err 0.56 m vs σ 0.40 m). Increase R_das in the Kalman noise matrix.

# 14. Per-Axis Error Breakdown (X vs Y)

Scalar RMSE hides directional bias.  A dominant Y error (lateral) usually points to insufficient lateral process noise (Q_yy too small) or missing sensors that constrain cross-track position.  A dominant X error suggests speed-model mismatch.

## Track T000001 — veh001

| Axis | Mean ¦error¦ (m) | RMSE (m) | Signed bias (m) | Dominant? |
| --- | --- | --- | --- | --- |
| X (longitudinal) | 0.220 | 0.278 | +0.004 | — |
| Y (lateral) | 0.269 | 0.341 | -0.000 | Yes |

✓ X and Y errors are balanced — no strong axis bias.

## Track T000002 — veh002

| Axis | Mean ¦error¦ (m) | RMSE (m) | Signed bias (m) | Dominant? |
| --- | --- | --- | --- | --- |
| X (longitudinal) | 0.203 | 0.256 | -0.002 | Yes |
| Y (lateral) | 0.200 | 0.253 | -0.001 | — |

✓ X and Y errors are balanced — no strong axis bias.

## Track T000003 — veh003

| Axis | Mean ¦error¦ (m) | RMSE (m) | Signed bias (m) | Dominant? |
| --- | --- | --- | --- | --- |
| X (longitudinal) | 0.201 | 0.256 | -0.013 | — |
| Y (lateral) | 0.266 | 0.334 | +0.008 | Yes |

✓ X and Y errors are balanced — no strong axis bias.

## Track T000004 — veh004

| Axis | Mean ¦error¦ (m) | RMSE (m) | Signed bias (m) | Dominant? |
| --- | --- | --- | --- | --- |
| X (longitudinal) | 0.194 | 0.284 | +0.009 | Yes |
| Y (lateral) | 0.124 | 0.174 | -0.018 | — |

⚠ X (longitudinal) error dominates by 1.6×. Consider increasing Q_xx (longitudinal process noise) or reviewing the vehicle speed model.

# 15. Sensor Coverage Timeline

Each row = one second of simulation.  ✓ = at least one accepted measurement;  — = sensor silent (filter coasted).  Seconds where ALL sensors are silent are the primary source of position drift.

## Track T000001 — veh001

| Second (s) | GPS | Camera | DAS | Status |
| --- | --- | --- | --- | --- |
| 0–1 | — | ✓ | ✓ | ✓ sensor active |
| 1–2 | — | — | ✓ | ✓ sensor active |
| 2–3 | ✓ | — | ✓ | ✓ sensor active |
| 3–4 | ✓ | — | ✓ | ✓ sensor active |
| 4–5 | ✓ | — | ✓ | ✓ sensor active |
| 5–6 | ✓ | — | ✓ | ✓ sensor active |
| 6–7 | ✓ | — | ✓ | ✓ sensor active |
| 7–8 | ✓ | — | ✓ | ✓ sensor active |
| 8–9 | ✓ | — | ✓ | ✓ sensor active |
| 9–10 | ✓ | — | ✓ | ✓ sensor active |
| 10–11 | ✓ | — | ✓ | ✓ sensor active |
| 11–12 | ✓ | — | ✓ | ✓ sensor active |
| 12–13 | ✓ | — | ✓ | ✓ sensor active |
| 13–14 | ✓ | — | ✓ | ✓ sensor active |
| 14–15 | ✓ | — | ✓ | ✓ sensor active |
| 15–16 | ✓ | — | ✓ | ✓ sensor active |
| 16–17 | ✓ | — | ✓ | ✓ sensor active |
| 17–18 | ✓ | — | ✓ | ✓ sensor active |
| 18–19 | ✓ | — | ✓ | ✓ sensor active |
| 19–20 | ✓ | — | ✓ | ✓ sensor active |
| 20–21 | ✓ | — | ✓ | ✓ sensor active |
| 21–22 | ✓ | — | ✓ | ✓ sensor active |
| 22–23 | ✓ | — | ✓ | ✓ sensor active |
| 23–24 | ✓ | — | ✓ | ✓ sensor active |
| 24–25 | ✓ | — | ✓ | ✓ sensor active |
| 25–26 | ✓ | — | ✓ | ✓ sensor active |
| 26–27 | ✓ | — | ✓ | ✓ sensor active |
| 27–28 | ✓ | — | ✓ | ✓ sensor active |
| 28–29 | ✓ | — | ✓ | ✓ sensor active |
| 29–30 | ✓ | — | ✓ | ✓ sensor active |
| 30–31 | ✓ | — | ✓ | ✓ sensor active |
| 31–32 | ✓ | — | ✓ | ✓ sensor active |
| 32–33 | ✓ | — | ✓ | ✓ sensor active |
| 33–34 | ✓ | — | ✓ | ✓ sensor active |
| 34–35 | ✓ | — | ✓ | ✓ sensor active |
| 35–36 | ✓ | — | ✓ | ✓ sensor active |
| 36–37 | ✓ | — | ✓ | ✓ sensor active |
| 37–38 | ✓ | — | ✓ | ✓ sensor active |
| 38–39 | ✓ | — | ✓ | ✓ sensor active |
| 39–40 | ✓ | — | ✓ | ✓ sensor active |
| 40–41 | ✓ | — | ✓ | ✓ sensor active |
| 41–42 | ✓ | — | ✓ | ✓ sensor active |
| 42–43 | ✓ | — | ✓ | ✓ sensor active |
| 43–44 | ✓ | — | ✓ | ✓ sensor active |
| 44–45 | ✓ | — | ✓ | ✓ sensor active |
| 45–46 | ✓ | — | ✓ | ✓ sensor active |
| 46–47 | ✓ | — | ✓ | ✓ sensor active |
| 47–48 | ✓ | — | ✓ | ✓ sensor active |
| 48–49 | ✓ | — | ✓ | ✓ sensor active |
| 49–50 | ✓ | — | ✓ | ✓ sensor active |
| 50–51 | ✓ | — | ✓ | ✓ sensor active |
| 51–52 | ✓ | — | ✓ | ✓ sensor active |
| 52–53 | ✓ | — | ✓ | ✓ sensor active |
| 53–54 | ✓ | — | ✓ | ✓ sensor active |
| 54–55 | ✓ | — | ✓ | ✓ sensor active |
| 55–56 | ✓ | — | ✓ | ✓ sensor active |
| 56–57 | ✓ | — | ✓ | ✓ sensor active |
| 57–58 | ✓ | — | ✓ | ✓ sensor active |
| 58–59 | ✓ | — | ✓ | ✓ sensor active |
| 59–60 | ✓ | — | ✓ | ✓ sensor active |
| 60–61 | ✓ | — | ✓ | ✓ sensor active |
| 61–62 | ✓ | — | ✓ | ✓ sensor active |
| 62–63 | ✓ | — | ✓ | ✓ sensor active |
| 63–64 | ✓ | — | ✓ | ✓ sensor active |
| 64–65 | ✓ | — | ✓ | ✓ sensor active |
| 65–66 | ✓ | — | ✓ | ✓ sensor active |
| 66–67 | ✓ | — | ✓ | ✓ sensor active |
| 67–68 | ✓ | — | ✓ | ✓ sensor active |
| 68–69 | ✓ | — | ✓ | ✓ sensor active |
| 69–70 | ✓ | — | ✓ | ✓ sensor active |
| 70–71 | ✓ | — | ✓ | ✓ sensor active |
| 71–72 | ✓ | — | ✓ | ✓ sensor active |
| 72–73 | ✓ | — | ✓ | ✓ sensor active |
| 73–74 | ✓ | — | ✓ | ✓ sensor active |
| 74–75 | ✓ | — | ✓ | ✓ sensor active |
| 75–76 | ✓ | — | ✓ | ✓ sensor active |
| 76–77 | ✓ | — | ✓ | ✓ sensor active |
| 77–78 | ✓ | — | ✓ | ✓ sensor active |
| 78–79 | ✓ | — | ✓ | ✓ sensor active |
| 79–80 | ✓ | — | ✓ | ✓ sensor active |
| 80–81 | ✓ | — | ✓ | ✓ sensor active |
| 81–82 | ✓ | — | ✓ | ✓ sensor active |
| 82–83 | ✓ | — | ✓ | ✓ sensor active |
| 83–84 | ✓ | — | ✓ | ✓ sensor active |
| 84–85 | ✓ | — | ✓ | ✓ sensor active |
| 85–86 | — | — | ✓ | ✓ sensor active |
| 86–87 | ✓ | — | ✓ | ✓ sensor active |
| 87–88 | ✓ | — | ✓ | ✓ sensor active |
| 88–89 | ✓ | — | ✓ | ✓ sensor active |
| 89–90 | ✓ | — | ✓ | ✓ sensor active |
| 90–91 | ✓ | — | ✓ | ✓ sensor active |
| 91–92 | ✓ | — | ✓ | ✓ sensor active |
| 92–93 | ✓ | — | ✓ | ✓ sensor active |
| 93–94 | ✓ | — | ✓ | ✓ sensor active |
| 94–95 | ✓ | — | ✓ | ✓ sensor active |
| 95–96 | ✓ | — | ✓ | ✓ sensor active |
| 96–97 | ✓ | — | ✓ | ✓ sensor active |
| 97–98 | ✓ | — | ✓ | ✓ sensor active |
| 98–99 | ✓ | — | ✓ | ✓ sensor active |
| 99–100 | ✓ | — | ✓ | ✓ sensor active |
| 100–101 | ✓ | — | ✓ | ✓ sensor active |
| 101–102 | ✓ | — | ✓ | ✓ sensor active |
| 102–103 | ✓ | — | ✓ | ✓ sensor active |
| 103–104 | ✓ | — | ✓ | ✓ sensor active |
| 104–105 | ✓ | — | ✓ | ✓ sensor active |
| 105–106 | ✓ | — | ✓ | ✓ sensor active |
| 106–107 | ✓ | — | ✓ | ✓ sensor active |
| 107–108 | ✓ | — | ✓ | ✓ sensor active |
| 108–109 | ✓ | — | ✓ | ✓ sensor active |
| 109–110 | ✓ | — | ✓ | ✓ sensor active |
| 110–111 | ✓ | — | ✓ | ✓ sensor active |
| 111–112 | ✓ | — | ✓ | ✓ sensor active |
| 112–113 | ✓ | — | ✓ | ✓ sensor active |
| 113–114 | ✓ | — | ✓ | ✓ sensor active |
| 114–115 | ✓ | — | ✓ | ✓ sensor active |
| 115–116 | ✓ | — | ✓ | ✓ sensor active |
| 116–117 | ✓ | — | ✓ | ✓ sensor active |
| 117–118 | ✓ | — | ✓ | ✓ sensor active |
| 118–119 | ✓ | — | ✓ | ✓ sensor active |
| 119–120 | ✓ | — | ✓ | ✓ sensor active |
| 120–121 | — | — | — | ✗ prediction only |

Dead seconds (all sensors silent): 1 / 121 (0%)

## Track T000002 — veh002

| Second (s) | GPS | Camera | DAS | Status |
| --- | --- | --- | --- | --- |
| 0–1 | ✓ | — | ✓ | ✓ sensor active |
| 1–2 | ✓ | — | ✓ | ✓ sensor active |
| 2–3 | ✓ | — | ✓ | ✓ sensor active |
| 3–4 | ✓ | — | ✓ | ✓ sensor active |
| 4–5 | ✓ | — | ✓ | ✓ sensor active |
| 5–6 | ✓ | — | ✓ | ✓ sensor active |
| 6–7 | ✓ | — | ✓ | ✓ sensor active |
| 7–8 | ✓ | — | ✓ | ✓ sensor active |
| 8–9 | ✓ | — | ✓ | ✓ sensor active |
| 9–10 | ✓ | — | ✓ | ✓ sensor active |
| 10–11 | ✓ | — | ✓ | ✓ sensor active |
| 11–12 | ✓ | — | ✓ | ✓ sensor active |
| 12–13 | ✓ | — | ✓ | ✓ sensor active |
| 13–14 | ✓ | — | ✓ | ✓ sensor active |
| 14–15 | ✓ | — | ✓ | ✓ sensor active |
| 15–16 | ✓ | — | ✓ | ✓ sensor active |
| 16–17 | ✓ | — | ✓ | ✓ sensor active |
| 17–18 | ✓ | — | ✓ | ✓ sensor active |
| 18–19 | ✓ | — | ✓ | ✓ sensor active |
| 19–20 | ✓ | — | ✓ | ✓ sensor active |
| 20–21 | ✓ | — | ✓ | ✓ sensor active |
| 21–22 | ✓ | — | ✓ | ✓ sensor active |
| 22–23 | ✓ | — | ✓ | ✓ sensor active |
| 23–24 | ✓ | — | ✓ | ✓ sensor active |
| 24–25 | ✓ | — | ✓ | ✓ sensor active |
| 25–26 | ✓ | — | ✓ | ✓ sensor active |
| 26–27 | ✓ | — | ✓ | ✓ sensor active |
| 27–28 | ✓ | — | ✓ | ✓ sensor active |
| 28–29 | ✓ | — | ✓ | ✓ sensor active |
| 29–30 | ✓ | — | ✓ | ✓ sensor active |
| 30–31 | ✓ | — | ✓ | ✓ sensor active |
| 31–32 | ✓ | — | ✓ | ✓ sensor active |
| 32–33 | ✓ | — | ✓ | ✓ sensor active |
| 33–34 | ✓ | — | ✓ | ✓ sensor active |
| 34–35 | ✓ | — | ✓ | ✓ sensor active |
| 35–36 | ✓ | — | ✓ | ✓ sensor active |
| 36–37 | ✓ | — | ✓ | ✓ sensor active |
| 37–38 | ✓ | — | ✓ | ✓ sensor active |
| 38–39 | ✓ | — | ✓ | ✓ sensor active |
| 39–40 | ✓ | — | ✓ | ✓ sensor active |
| 40–41 | ✓ | — | ✓ | ✓ sensor active |
| 41–42 | ✓ | — | ✓ | ✓ sensor active |
| 42–43 | ✓ | — | ✓ | ✓ sensor active |
| 43–44 | ✓ | — | ✓ | ✓ sensor active |
| 44–45 | ✓ | — | ✓ | ✓ sensor active |
| 45–46 | ✓ | — | ✓ | ✓ sensor active |
| 46–47 | ✓ | — | ✓ | ✓ sensor active |
| 47–48 | ✓ | — | ✓ | ✓ sensor active |
| 48–49 | ✓ | — | ✓ | ✓ sensor active |
| 49–50 | ✓ | — | ✓ | ✓ sensor active |
| 50–51 | ✓ | — | ✓ | ✓ sensor active |
| 51–52 | ✓ | — | ✓ | ✓ sensor active |
| 52–53 | ✓ | — | ✓ | ✓ sensor active |
| 53–54 | ✓ | — | ✓ | ✓ sensor active |
| 54–55 | ✓ | — | ✓ | ✓ sensor active |
| 55–56 | ✓ | — | ✓ | ✓ sensor active |
| 56–57 | ✓ | — | ✓ | ✓ sensor active |
| 57–58 | ✓ | — | ✓ | ✓ sensor active |
| 58–59 | ✓ | — | ✓ | ✓ sensor active |
| 59–60 | ✓ | — | ✓ | ✓ sensor active |
| 60–61 | ✓ | — | ✓ | ✓ sensor active |
| 61–62 | ✓ | — | ✓ | ✓ sensor active |
| 62–63 | ✓ | — | ✓ | ✓ sensor active |
| 63–64 | ✓ | — | ✓ | ✓ sensor active |
| 64–65 | ✓ | — | ✓ | ✓ sensor active |
| 65–66 | ✓ | — | ✓ | ✓ sensor active |
| 66–67 | ✓ | — | ✓ | ✓ sensor active |
| 67–68 | ✓ | — | ✓ | ✓ sensor active |
| 68–69 | ✓ | — | ✓ | ✓ sensor active |
| 69–70 | ✓ | — | ✓ | ✓ sensor active |
| 70–71 | ✓ | — | ✓ | ✓ sensor active |
| 71–72 | ✓ | — | ✓ | ✓ sensor active |
| 72–73 | ✓ | — | ✓ | ✓ sensor active |
| 73–74 | ✓ | — | ✓ | ✓ sensor active |
| 74–75 | ✓ | — | ✓ | ✓ sensor active |
| 75–76 | ✓ | — | ✓ | ✓ sensor active |
| 76–77 | ✓ | — | ✓ | ✓ sensor active |
| 77–78 | ✓ | — | ✓ | ✓ sensor active |
| 78–79 | ✓ | — | ✓ | ✓ sensor active |
| 79–80 | ✓ | — | ✓ | ✓ sensor active |
| 80–81 | ✓ | — | ✓ | ✓ sensor active |
| 81–82 | ✓ | — | ✓ | ✓ sensor active |
| 82–83 | ✓ | — | ✓ | ✓ sensor active |
| 83–84 | ✓ | — | ✓ | ✓ sensor active |
| 84–85 | ✓ | — | ✓ | ✓ sensor active |
| 85–86 | — | — | ✓ | ✓ sensor active |
| 86–87 | ✓ | — | ✓ | ✓ sensor active |
| 87–88 | ✓ | — | ✓ | ✓ sensor active |
| 88–89 | ✓ | — | ✓ | ✓ sensor active |
| 89–90 | ✓ | — | ✓ | ✓ sensor active |
| 90–91 | ✓ | — | ✓ | ✓ sensor active |
| 91–92 | ✓ | — | ✓ | ✓ sensor active |
| 92–93 | ✓ | — | ✓ | ✓ sensor active |
| 93–94 | ✓ | — | ✓ | ✓ sensor active |
| 94–95 | ✓ | — | ✓ | ✓ sensor active |
| 95–96 | ✓ | — | ✓ | ✓ sensor active |
| 96–97 | ✓ | — | ✓ | ✓ sensor active |
| 97–98 | ✓ | — | ✓ | ✓ sensor active |
| 98–99 | ✓ | — | ✓ | ✓ sensor active |
| 99–100 | ✓ | — | ✓ | ✓ sensor active |
| 100–101 | ✓ | — | ✓ | ✓ sensor active |
| 101–102 | ✓ | — | ✓ | ✓ sensor active |
| 102–103 | ✓ | — | ✓ | ✓ sensor active |
| 103–104 | ✓ | — | ✓ | ✓ sensor active |
| 104–105 | ✓ | — | ✓ | ✓ sensor active |
| 105–106 | ✓ | — | ✓ | ✓ sensor active |
| 106–107 | ✓ | — | ✓ | ✓ sensor active |
| 107–108 | ✓ | — | ✓ | ✓ sensor active |
| 108–109 | ✓ | — | ✓ | ✓ sensor active |
| 109–110 | ✓ | — | ✓ | ✓ sensor active |
| 110–111 | ✓ | — | ✓ | ✓ sensor active |
| 111–112 | ✓ | — | ✓ | ✓ sensor active |
| 112–113 | ✓ | — | ✓ | ✓ sensor active |
| 113–114 | ✓ | — | ✓ | ✓ sensor active |
| 114–115 | ✓ | — | ✓ | ✓ sensor active |
| 115–116 | ✓ | — | ✓ | ✓ sensor active |
| 116–117 | ✓ | — | ✓ | ✓ sensor active |
| 117–118 | ✓ | — | ✓ | ✓ sensor active |
| 118–119 | ✓ | — | ✓ | ✓ sensor active |
| 119–120 | ✓ | — | ✓ | ✓ sensor active |
| 120–121 | — | — | — | ✗ prediction only |

Dead seconds (all sensors silent): 1 / 121 (0%)

## Track T000003 — veh003

| Second (s) | GPS | Camera | DAS | Status |
| --- | --- | --- | --- | --- |
| 0–1 | ✓ | — | ✓ | ✓ sensor active |
| 1–2 | ✓ | — | ✓ | ✓ sensor active |
| 2–3 | — | — | ✓ | ✓ sensor active |
| 3–4 | — | — | ✓ | ✓ sensor active |
| 4–5 | — | — | ✓ | ✓ sensor active |
| 5–6 | ✓ | — | ✓ | ✓ sensor active |
| 6–7 | ✓ | — | ✓ | ✓ sensor active |
| 7–8 | ✓ | — | ✓ | ✓ sensor active |
| 8–9 | ✓ | — | ✓ | ✓ sensor active |
| 9–10 | — | ✓ | ✓ | ✓ sensor active |
| 10–11 | — | ✓ | ✓ | ✓ sensor active |
| 11–12 | ✓ | ✓ | ✓ | ✓ sensor active |
| 12–13 | ✓ | ✓ | ✓ | ✓ sensor active |
| 13–14 | ✓ | ✓ | ✓ | ✓ sensor active |
| 14–15 | ✓ | ✓ | ✓ | ✓ sensor active |
| 15–16 | ✓ | — | ✓ | ✓ sensor active |
| 16–17 | — | — | ✓ | ✓ sensor active |
| 17–18 | — | — | ✓ | ✓ sensor active |
| 18–19 | — | — | ✓ | ✓ sensor active |
| 19–20 | ✓ | — | ✓ | ✓ sensor active |
| 20–21 | ✓ | — | ✓ | ✓ sensor active |
| 21–22 | ✓ | — | ✓ | ✓ sensor active |
| 22–23 | ✓ | — | ✓ | ✓ sensor active |
| 23–24 | ✓ | — | ✓ | ✓ sensor active |
| 24–25 | ✓ | — | ✓ | ✓ sensor active |
| 25–26 | ✓ | — | ✓ | ✓ sensor active |
| 26–27 | ✓ | — | ✓ | ✓ sensor active |
| 27–28 | ✓ | — | ✓ | ✓ sensor active |
| 28–29 | ✓ | — | ✓ | ✓ sensor active |
| 29–30 | ✓ | — | ✓ | ✓ sensor active |
| 30–31 | ✓ | — | ✓ | ✓ sensor active |
| 31–32 | ✓ | — | ✓ | ✓ sensor active |
| 32–33 | ✓ | — | ✓ | ✓ sensor active |
| 33–34 | ✓ | — | ✓ | ✓ sensor active |
| 34–35 | ✓ | — | ✓ | ✓ sensor active |
| 35–36 | ✓ | — | ✓ | ✓ sensor active |
| 36–37 | ✓ | — | ✓ | ✓ sensor active |
| 37–38 | ✓ | — | ✓ | ✓ sensor active |
| 38–39 | ✓ | — | ✓ | ✓ sensor active |
| 39–40 | ✓ | — | ✓ | ✓ sensor active |
| 40–41 | ✓ | — | ✓ | ✓ sensor active |
| 41–42 | ✓ | — | ✓ | ✓ sensor active |
| 42–43 | ✓ | — | ✓ | ✓ sensor active |
| 43–44 | ✓ | — | ✓ | ✓ sensor active |
| 44–45 | ✓ | — | ✓ | ✓ sensor active |
| 45–46 | ✓ | — | ✓ | ✓ sensor active |
| 46–47 | ✓ | — | ✓ | ✓ sensor active |
| 47–48 | ✓ | — | ✓ | ✓ sensor active |
| 48–49 | ✓ | — | ✓ | ✓ sensor active |
| 49–50 | ✓ | — | ✓ | ✓ sensor active |
| 50–51 | ✓ | — | ✓ | ✓ sensor active |
| 51–52 | ✓ | — | ✓ | ✓ sensor active |
| 52–53 | ✓ | — | ✓ | ✓ sensor active |
| 53–54 | ✓ | — | ✓ | ✓ sensor active |
| 54–55 | ✓ | — | ✓ | ✓ sensor active |
| 55–56 | ✓ | — | ✓ | ✓ sensor active |
| 56–57 | ✓ | — | ✓ | ✓ sensor active |
| 57–58 | ✓ | — | ✓ | ✓ sensor active |
| 58–59 | ✓ | — | ✓ | ✓ sensor active |
| 59–60 | ✓ | — | ✓ | ✓ sensor active |
| 60–61 | ✓ | — | ✓ | ✓ sensor active |
| 61–62 | ✓ | — | ✓ | ✓ sensor active |
| 62–63 | ✓ | — | ✓ | ✓ sensor active |
| 63–64 | ✓ | — | ✓ | ✓ sensor active |
| 64–65 | ✓ | — | ✓ | ✓ sensor active |
| 65–66 | ✓ | — | ✓ | ✓ sensor active |
| 66–67 | ✓ | — | ✓ | ✓ sensor active |
| 67–68 | ✓ | — | ✓ | ✓ sensor active |
| 68–69 | ✓ | — | ✓ | ✓ sensor active |
| 69–70 | ✓ | — | ✓ | ✓ sensor active |
| 70–71 | ✓ | — | ✓ | ✓ sensor active |
| 71–72 | ✓ | — | ✓ | ✓ sensor active |
| 72–73 | ✓ | — | ✓ | ✓ sensor active |
| 73–74 | ✓ | — | ✓ | ✓ sensor active |
| 74–75 | ✓ | — | ✓ | ✓ sensor active |
| 75–76 | ✓ | — | ✓ | ✓ sensor active |
| 76–77 | ✓ | — | ✓ | ✓ sensor active |
| 77–78 | ✓ | — | ✓ | ✓ sensor active |
| 78–79 | ✓ | — | ✓ | ✓ sensor active |
| 79–80 | ✓ | — | ✓ | ✓ sensor active |
| 80–81 | ✓ | — | ✓ | ✓ sensor active |
| 81–82 | ✓ | — | ✓ | ✓ sensor active |
| 82–83 | ✓ | — | ✓ | ✓ sensor active |
| 83–84 | ✓ | — | ✓ | ✓ sensor active |
| 84–85 | ✓ | — | ✓ | ✓ sensor active |
| 85–86 | — | — | ✓ | ✓ sensor active |
| 86–87 | ✓ | — | ✓ | ✓ sensor active |
| 87–88 | ✓ | — | ✓ | ✓ sensor active |
| 88–89 | ✓ | — | ✓ | ✓ sensor active |
| 89–90 | ✓ | — | ✓ | ✓ sensor active |
| 90–91 | ✓ | — | ✓ | ✓ sensor active |
| 91–92 | ✓ | — | ✓ | ✓ sensor active |
| 92–93 | ✓ | — | ✓ | ✓ sensor active |
| 93–94 | ✓ | — | ✓ | ✓ sensor active |
| 94–95 | ✓ | — | ✓ | ✓ sensor active |
| 95–96 | ✓ | — | ✓ | ✓ sensor active |
| 96–97 | ✓ | — | ✓ | ✓ sensor active |
| 97–98 | ✓ | — | ✓ | ✓ sensor active |
| 98–99 | ✓ | — | ✓ | ✓ sensor active |
| 99–100 | ✓ | — | ✓ | ✓ sensor active |
| 100–101 | ✓ | — | ✓ | ✓ sensor active |
| 101–102 | ✓ | — | ✓ | ✓ sensor active |
| 102–103 | ✓ | — | ✓ | ✓ sensor active |
| 103–104 | ✓ | — | ✓ | ✓ sensor active |
| 104–105 | ✓ | — | ✓ | ✓ sensor active |
| 105–106 | ✓ | — | ✓ | ✓ sensor active |
| 106–107 | ✓ | — | ✓ | ✓ sensor active |
| 107–108 | ✓ | — | ✓ | ✓ sensor active |
| 108–109 | ✓ | — | ✓ | ✓ sensor active |
| 109–110 | ✓ | — | ✓ | ✓ sensor active |
| 110–111 | ✓ | — | ✓ | ✓ sensor active |
| 111–112 | ✓ | — | ✓ | ✓ sensor active |
| 112–113 | ✓ | — | ✓ | ✓ sensor active |
| 113–114 | ✓ | — | ✓ | ✓ sensor active |
| 114–115 | ✓ | — | ✓ | ✓ sensor active |
| 115–116 | ✓ | — | ✓ | ✓ sensor active |
| 116–117 | ✓ | — | ✓ | ✓ sensor active |
| 117–118 | ✓ | — | ✓ | ✓ sensor active |
| 118–119 | ✓ | — | ✓ | ✓ sensor active |
| 119–120 | ✓ | — | ✓ | ✓ sensor active |
| 120–121 | — | — | — | ✗ prediction only |

Dead seconds (all sensors silent): 1 / 121 (0%)

## Track T000004 — veh004

| Second (s) | GPS | Camera | DAS | Status |
| --- | --- | --- | --- | --- |
| 0–1 | ✓ | ✓ | ✓ | ✓ sensor active |
| 1–2 | ✓ | ✓ | ✓ | ✓ sensor active |
| 2–3 | ✓ | ✓ | ✓ | ✓ sensor active |
| 3–4 | ✓ | ✓ | ✓ | ✓ sensor active |
| 4–5 | ✓ | ✓ | ✓ | ✓ sensor active |
| 5–6 | ✓ | ✓ | ✓ | ✓ sensor active |
| 6–7 | ✓ | ✓ | ✓ | ✓ sensor active |
| 7–8 | ✓ | ✓ | ✓ | ✓ sensor active |
| 8–9 | ✓ | ✓ | ✓ | ✓ sensor active |
| 9–10 | ✓ | ✓ | ✓ | ✓ sensor active |
| 10–11 | ✓ | ✓ | ✓ | ✓ sensor active |
| 11–12 | — | ✓ | ✓ | ✓ sensor active |
| 12–13 | — | ✓ | ✓ | ✓ sensor active |
| 13–14 | — | — | — | ✗ prediction only |

Dead seconds (all sensors silent): 1 / 14 (7%)

# 16. Cold-Start Latency

Time from the first trajectory row to the first moment the position error drops below 0.5 m.  A long cold-start indicates the filter is initialized far from the true position or the initial covariance P₀ is too small.

| Track | Vehicle | Track start (s) | First error < 0.5 m at (s) | Cold-start latency (s) | Initial error (m) |
| --- | --- | --- | --- | --- | --- |
| T000001 | veh001 | 0.020 | 0.540 | 0.520 s | 0.854 |
| T000002 | veh002 | 0.020 | 0.020 | 0.000 s | 0.309 |
| T000003 | veh003 | 0.020 | 0.020 | 0.000 s | 0.403 |
| T000004 | veh004 | 0.020 | 0.020 | 0.000 s | 0.054 |

# 17. Speed vs Position Error

Position error bucketed by estimated vehicle speed.  If error grows with speed, the process noise Q or motion model is misspecified for high-speed dynamics.  If error is flat, sensor dropout is the dominant driver.

| Speed bucket | N rows | Mean error (m) | Max error (m) | Trend vs prev |
| --- | --- | --- | --- | --- |
| 0–5 m/s | 16351 | 0.354 | 1.852 | — |
| 5–10 m/s | 1167 | 0.394 | 2.115 | ↑ growing |
| 10–15 m/s | 1068 | 0.280 | 2.789 | ↓ shrinking |
| 15–20 m/s | 41 | 0.462 | 0.788 | ↑ growing |
| 20+ m/s | 3 | 0.624 | 0.711 | ↑ growing |

⚠ Error grows from 0.35 m (at 0–5 m/s) to 0.62 m (at 20+ m/s) — process noise Q likely needs to be increased for higher speeds, especially the acceleration term.