SimStudio

Physical Models Reference

Version: March 2026  •  Complete model documentation + today's changes

# 1.  Vehicle Dynamics

Each vehicle is a point mass on a lane polyline. The simulator advances kinematics with a time-stepped first-order model, stochastic speed changes, and lateral OU-drift.

## 1.1  Longitudinal Model

Speed tracking uses a first-order lag controller. At each tick the desired acceleration command is:

a_cmd = (v_target − v) / τ_accel       τ_accel = 1.5 s

The command is clipped to the per-vehicle acceleration limits, then velocity and arc-length position are integrated with the trapezoidal rule:

v_{k+1} = v_k + a_cmd · dt           s_{k+1} = s_k + ½(v_k + v_{k+1}) · dt

- Deadband: command set to 0 when |Δv| < 0.12 m/s (prevents micro-oscillation)

- Cruise hold: command set to 0 when |Δv| < 0.18 m/s (visible constant-speed periods)

- Hard speed cap: 1.12 × speed_limit or 4 m/s minimum

## 1.2  Stochastic Speed Profile

Each vehicle draws a macro speed profile at spawn. Target speed is resampled every speed_change_interval_mean_s seconds from a Gaussian, then the controller ramps toward it.

- speed_mean_mps, speed_std_mps: lane-speed-based distribution

- cruise_hold_probability: probability that a resample keeps current target (no change)

- max_accel_mps2 / max_decel_mps2: per-vehicle physical bounds

## 1.3  Lateral Drift (Ornstein–Uhlenbeck)

Lateral offset from lane centreline follows a discrete-time OU process — mean-reverting stochastic drift bounded to ± ½ lane_width:

lat_{k+1} = ρ · lat_k + N(0, σ_lat · √(1−ρ²))     ρ = e^(−dt/τ_lat)

- τ_lat = 1.5 s  (mean-reversion time constant)

- σ_lat = 0.5 × max_lat  (drift standard deviation)

- Clipped to [−½ lane_width, +½ lane_width] after each step

Lateral offset affects all sensor geometries: GPS range, camera angle, and DAS fiber distance r.

# 2.  Sensor Models

## 2.1  GPS Sensor

Emits noisy (x, y) position for every vehicle within its radius_m. Noise grows with distance:

σ_gps = max(0.20,  σ₀ · (1 + 0.35 · (r / r_max)²))

confidence = 1 / (1 + (σ_gps / σ₀)²)

- σ₀ = sigma_m (configured per sensor, default 2.5 m)

- Noise applied independently in x and y: N(0, σ_gps)

- No detection gating — GPS reports all vehicles in radius unconditionally

## 2.2  Camera Sensor

Pinhole-projection model. Noise has three components: a fixed floor, a range term, and an off-axis distortion term.

σ_cam = max(0.08,  σ₀ + k_r · r + k_θ · |θ|)

Constants:

| Symbol | Value | Units | Meaning |
| --- | --- | --- | --- |
| σ₀ | 0.10 | m | Irreducible floor (quantisation + calibration) |
| k_r | σ_px / focal_px | m/m | Pinhole pixel-error → metres (σ_px = 1.5 px, W_px = 1280) |
| k_θ | 0.15 | m/rad | Off-axis geometric distortion |

Detection probability (stochastic gating):

conf = 0.15 + 0.83 · (1 − r/r_max)^1.35 · (0.55 + 0.45 · (1 − |θ|/θ_max))

A frame is skipped if random() > conf.  This models partial occlusion and low-confidence detections.

## 2.3  DAS Sensor  (Distributed Acoustic Sensing)

DAS measures ground-borne vibration along an optical fiber buried beside the road. The simulator models the full chain from vehicle weight to Kalman measurement, now including detection gating, systematic position bias, and multi-vehicle peak merging.

### 2.3.1  Amplitude Model (Flamant–Boussinesq)

The DAS strain amplitude at the fiber due to a surface load W (vehicle weight, N) at perpendicular distance r is:

A = W / (r + d₀)²

- W = weight_kg (vehicle weight in kg, used as proxy for ground force)

- r = perpendicular distance from vehicle to nearest point on fiber polyline (m)

- d₀ = d0_m = 0.7 m (singularity floor — prevents A → ∞ at r = 0)

The trace is built by spreading each vehicle's amplitude into neighbouring channels with a Gaussian kernel (σ² = 9 channels), then adding white noise:

trace[j] += exp(−k² / 18) · A       for k ∈ [−8, +8]

trace_noisy[j] = trace[j] + N(0, noise_std)

### 2.3.2  SNR and Detection Probability  ★ updated today

Signal-to-noise ratio at each vehicle's fiber position:

SNR = A / noise_std

Detection probability — sigmoid in SNR² space (NEW):

p_detect = SNR² / (SNR² + t²)         t = snr_det_threshold  (default 2.0)

Interpretation: at SNR = t the vehicle is detected 50% of the time.  Missed detections publish a lightweight sensor.das_miss event (snr, p_detect, fiber_distance_m) instead of sensor.das.

| Scenario | Weight | r (m) | SNR | p_detect |
| --- | --- | --- | --- | --- |
| Standard car | 1500 kg | 2 | 6.9 | 92 % |
| Motorcycle | 600 kg | 2 | 2.7 | 64 % |
| Bus | 12 000 kg | 2 | 54.9 | 99.9 % |
| Car, far distance | 1500 kg | 10 | 0.4 | 9 % |

### 2.3.3  Position Measurement with Systematic Bias  ★ updated today

For detected vehicles, the measured fiber arc-length s_meas is drawn from:

s_noisy  =  s_fiber  +  N(0, σ_x)  +  β

σ_x = max(0.8,  k_DAS / √SNR)         k_DAS = 2.0 m

β = ± bias_k / SNR    (random sign each tick)    bias_k = 0.5 m  (default)

- σ_x: Gaussian component — noise floor 0.8 m, shrinks as SNR improves

- β: systematic centroid shift — mimics peak-finding bias when signal is partly buried; magnitude is 0.07 m at SNR = 7, 0.25 m at SNR = 2

- Random sign per tick: not correlated across time — adds effective noise without a fixed drift

The world (x, y) position is recovered by mapping s_meas back onto the fiber polyline.  The σ_x value is forwarded to the Kalman filter as the DAS measurement noise std.

### 2.3.4  Multi-Vehicle Peak Merging  ★ new today

When two vehicles are close along the fiber, their strain peaks overlap and the peak detector either merges them into one measurement or shifts their centroids toward each other.

d_sep = |s_meas_A − s_meas_B|

| Condition | Outcome | Details |
| --- | --- | --- |
| d_sep < threshold/2 | Full merge | Amplitude-weighted centroid; dominant vehicle (higher A) keeps its ID; weaker is dropped; σ_x × 1.5 |
| d_sep < threshold | Partial attraction | Each centroid pulled 40% toward the other, weighted by relative amplitude; both measurements kept |
| d_sep ≥ threshold | No interaction | Measurements are independent (clean) |

Default threshold: 10 m.  Set peak_merge_threshold_m = 0 to disable.  Merge state (clean / merged / attracted) is logged in every sensor.das payload.

### 2.3.5  Velocity Measurement

DAS measures the along-fiber component of vehicle speed.  The signed scalar z_fiber is computed from the vehicle's lane heading and the fiber tangent angle θ:

z_fiber = v_true · cos(heading_road − θ_fiber)

σ_v = max(0.10,  k_v / √SNR)              k_v = 3.0 m/s

- Speed noise added: v_meas = v_true + N(0, speed_sigma),  speed_sigma = max(0.6, 4/SNR)

- σ_v is computed from SNR and forwarded to the Kalman filter (replaces the old fixed 1.20 m/s floor — see Section 4)

- Heading convention: fiber tangent direction is positive; opposite direction uses fiber_angle + π

# 3.  Kalman Filter

## 3.1  State and Prediction

6-state Constant Acceleration (CA) model with decoupled x and y axes:

x = [x,  vx,  ax,  y,  vy,  ay]ᵀ

x_{k+1} = F · x_k          F = block-diag(F_axis, F_axis)

F_axis = [[1, dt, dt²/2], [0, 1, dt], [0, 0, 1]]

Process noise covariance (singer noise model, σ_q = 2.0 m/s²):

Q = σ_q² · G · Gᵀ

G_axis = [dt²/2, dt, 1]ᵀ

## 3.2  Measurement Updates

Three separate update methods are used depending on sensor type:

| Sensor | Method | Observation matrix H | Notes |
| --- | --- | --- | --- |
| DAS position | update_position_x_only | H = [1, 0, 0, 0, 0, 0] | y-state intentionally unaffected — DAS cannot observe y |
| DAS velocity | update_velocity_fiber | H = [0, cos θ, 0, 0, sin θ, 0] | θ = local fiber tangent angle; geometrically correct for any road angle |
| Camera / GPS | update_positions / update_velocities | H_pos = [1,0,0,0,0,0; 0,0,0,1,0,0] | Full (x, y) position and (vx, vy) velocity |

All updates use the Joseph-form covariance equation for numerical stability:

P_{k|k} = (I − KH) · P_{k|k−1} · (I − KH)ᵀ  +  K · R · Kᵀ

## 3.3  Dynamic Measurement Noise R

Kalman R matrices are set from the SNR-derived sigma values forwarded in each sensor event, not from fixed lookup tables:

R_x^DAS = σ_x²    where   σ_x = max(0.8,  2.0 / √SNR)

R_v^DAS = σ_v²    where   σ_v = max(0.10, 3.0 / √SNR)

R_x^GPS    = σ_gps²       R_x^cam = σ_cam²

- Confidence value from the sensor event further scales R: σ_eff = σ / √conf

- Low-confidence measurements automatically receive less weight without any manual tuning

## 3.4  State Initialisation

On first measurement for a new vehicle track:

- If ground-truth is available (playback mode): seed from truth position; vx/vy from first measurement's speed and heading

- DAS-only, no truth: x = s_meas mapped back to world frame, y = 0.0 (road centreline — not fiber_offset_m, see Section 4)

- Camera / GPS, no truth: x, y = first measurement position

- Initial covariance P₀ = diag([pos_σ², vel_σ², acc_σ², pos_σ², vel_σ², acc_σ²])  with pos_σ = 2.5 m, vel_σ = 2.0 m/s, acc_σ = 1.0 m/s²

# 4.  What Changed Today vs. Previous Version

Summary of all changes made in today's session, grouped by impact area.  The previous version refers to the codebase before this session began.

| Area | Previous version | Today's version |
| --- | --- | --- |
| DAS track initialisation (y) | y seeded from seed.y = fiber_offset_m ≈ 2 m → permanent 2 m y-bias on DAS-only tracks | y = 0.0 (road centreline). DAS cannot observe y, so we no longer seed it from the fiber position. |
| DAS velocity sigma σ_v | Fixed floor 1.20 m/s for ALL DAS measurements (σ_v = max(1.20, 0.60 × σ_x)). Dominated at every practical SNR. | σ_v = max(0.10, 3.0 / √SNR). Dynamic: 1.14 m/s for a car, 0.40 m/s for a bus, 4.7 m/s for a car at 10 m. |
| DAS velocity geometry | H = [0,1,0,0,0,0] — assumed fiber along x-axis (θ = 0 hardcoded) | H = [0, cos θ, 0, 0, sin θ, 0] — geometrically correct for any fiber angle via update_velocity_fiber |
| DAS detection probability | Every vehicle on the segment always produces a measurement — no gating, no misses | p_detect = SNR² / (SNR² + t²). Misses publish sensor.das_miss. Motorcycle at 2 m: 64% detection rate. |
| DAS position noise model | Pure Gaussian: N(0, σ_x). No systematic term. | Gaussian + systematic bias: β = ± bias_k/SNR (random sign per tick). Mimics centroid-pull when peak is partly buried. |
| Multi-vehicle DAS ambiguity | Perfect independence: every vehicle always gets its own clean measurement regardless of proximity | Peak merging: full merge (<threshold/2) or 40% centroid attraction (<threshold). merge_state field logged. |
| Angled fiber scenarios | No angled-fiber test cases existed. All test roads were horizontal. | 3 new scenarios: 15°, 30°, 45°. Validate that update_velocity_fiber correctly projects vx, vy from z_fiber. |
| File load error handling | _load_scene had no try/except. Tkinter silently swallowed exceptions → GUI appeared frozen/stuck | Full try/except with rollback to previous world, messagebox showing the exact error + common causes list. |

## 4.1  What Was NOT Changed (intentionally kept)

- Kalman 6-state CA model structure — unchanged

- Joseph-form update — unchanged

- GPS and camera sensor models — unchanged

- Flamant-Boussinesq amplitude formula A = W/(r+d₀)² — unchanged

- Data association — still perfect per-vehicle (each vehicle has its own event ID)

- No full signal/waterfall simulation — intentionally out of scope

# 5.  Quick Reference — All Formulas

| Model | Formula | Constants / Parameters |
| --- | --- | --- |
| DAS Amplitude | A = W / (r + d₀)² | — | Flamant-Boussinesq |
| SNR | SNR = A / noise_std | noise_std = 30 (default) | noise floor |
| Detection p | p = SNR² / (SNR² + t²) | t = snr_det_threshold = 2.0 | sigmoid gate |
| DAS σ_x | σ_x = max(0.8, 2.0/√SNR) | floor 0.8 m | position noise |
| DAS bias β | β = ± bias_k / SNR | bias_k = 0.5 m | centroid shift |
| DAS σ_v | σ_v = max(0.10, 3.0/√SNR) | floor 0.10 m/s | velocity noise |
| DAS z_fiber | z = v·cos(θ_road − θ_fiber) | θ = local tangent | along-fiber speed |
| DAS H (pos) | H = [1,0,0,0,0,0] | x only | y not observed |
| DAS H (vel) | H = [0,cosθ,0,0,sinθ,0] | θ = fiber angle | geometrically correct |
| GPS σ | σ = max(0.20, σ₀·(1+0.35·(r/R)²)) | σ₀ = 2.5 m, R = radius | range noise |
| Camera σ | σ = max(0.08, 0.10 + k_r·r + 0.15¦θ¦) | k_r = 1.5/focal_px | pinhole model |
| Camera conf | c = 0.15+0.83·(1−r/R)^1.35·(0.55+0.45·a) | a = angle factor | detection gate |
| OU lateral | lat_{k+1} = ρ·lat_k + N(0,σ_lat√(1−ρ²)) | ρ = exp(−dt/1.5) | lane drift |
| Speed tracking | a = (v_target−v)/1.5 | clipped to [−a_max, a_max] | longitudinal |

SimStudio Physical Models Reference  •  Generated March 2026