SimStudio — Fiber Optic Traffic Lab

Complete Equations Reference  ·  Stage B v063  ·  March 2026

All mathematical formulas used by the simulator, rendered using Word's native equation editor. Variables in italics, operators and numbers in upright type. Units are SI throughout.

1   Vehicle Longitudinal Dynamics

Each vehicle tracks a stochastic target speed via a first-order controller. Dynamics are integrated once per simulation tick of duration dt.

1.1  Speed Error

Speed error

Δv = vtarget − vcurrent

1.2  Acceleration Command

Deadband: if |Δv| < 0.12 m/s then a_cmd = 0. Otherwise:

Acceleration command

acmd = Δvτ

where τ = 1.5 s  (ACCEL_RESPONSE_TAU_S).

Clamp to physical limits

anow = clampacmd,  −adecel,  +aaccel

Cruise hold: if |Δv| < 0.18 m/s then a_now = 0.

1.3  Speed and Position Integration  (trapezoidal Euler)

Speed update

vnext = clampvprev + anow · dt,  0,  vcap

Arc-length update

s ← s + 12vprev + vnext · dt

1.4  World-Frame Position, Heading and Acceleration

Position from arc-length

x, y = point_at_slane polyline,  s + λ · ñ̂

where λ is the lateral offset and n̂ is the left-perpendicular unit normal to the lane tangent.

Heading

θ = atan2ty, tx

where (t_x, t_y) is the unit tangent of the lane polyline at arc-length s.

World-frame acceleration components

ax = along · cosθay = along · sinθ

| Symbol | Value / Units | Description |
| --- | --- | --- |
| Δv | m/s | Speed error = v_target − v_current |
| τ | 1.5 s | First-order time constant |
| a_cmd | m/s² | Commanded acceleration before clamping |
| a_accel | 1.8 m/s² | Maximum acceleration (default) |
| a_decel | 2.6 m/s² | Maximum deceleration (default) |
| dt | s | Simulation tick duration |
| s | m | Arc-length along lane polyline from lane start |
| θ | rad | Lane tangent heading (world frame) |

2   Lateral Lane-Drift  (Ornstein–Uhlenbeck Process)

Vehicles drift within their lane following a discrete OU process with zero mean (lane centreline) and reversion time constant τ_lat = 1.5 s.

Mean-reversion factor

ρ = exp−dtτlat

Per-step noise standard deviation

σstep = σlat · 1 − ρ2

where σ_lat = lane_width / 4  (half the maximum lateral offset).

OU update rule

λnew = ρ · λold + ℔0, σstep

Hard clamp to lane boundary

λnew = clampλnew,  −W2,  +W2

W = lane_width = 3.6 m  (default).

| Symbol | Value / Units | Description |
| --- | --- | --- |
| τ_lat | 1.5 s | OU reversion time constant (LATERAL_DRIFT_TAU_S) |
| ρ | — | Discrete reversion factor ∈ [0, 1) |
| σ_lat | m | Noise scale = lane_width / 4 |
| λ | m | Signed lateral offset from centreline (+ = left) |
| W | 3.6 m | Lane width |

3   GPS Sensor

Fixed receiver that observes any vehicle within its coverage radius. Position noise grows with range; speed and acceleration are reported noise-free.

3.1  Range and Visibility

Euclidean range to vehicle

r = xtrue − xGPS2 + ytrue − yGPS2

Vehicle is observed only when  r ≤ radius_m  (default 25 m).

3.2  Position Noise Model

Range-dependent effective sigma

σ = max0.20,  σbase · 1 + 0.35 · rrmax2

Position measurement

xmeas = xtrue + nx,          nx ∼ ℔0, σ2ymeas = ytrue + ny,          ny ∼ ℔0, σ2

Confidence score

confidence = clamp11 + σσbase2,  0.05,  0.99

3.3  Speed and Acceleration Output  (no noise)

Speed output

speed_mps = vtrue

Accel output

a_long = along, true

| Symbol | Value / Units | Description |
| --- | --- | --- |
| σ_base | m | Baseline noise std (GPSSensor.sigma_m, default 2.5 m) |
| r_max | m | Coverage radius (default 25 m) |
| σ | m | Effective noise std at distance r |
| confidence | — | Quality score fed to Kalman weighting ∈ [0.05, 0.99] |

4   Camera Sensor

Pinhole model with stochastic detection. Noise grows with range and off-axis angle. Detection probability equals the confidence score.

4.1  Visibility Check

Range

r = xtrue − xcam2 + ytrue − ycam2

Bearing and off-axis angle

α = atan2ytrue − ycam, xtrue − xcamΔα = wrap_angleα − θcam

Visible if  r ≤ range_m  AND  |Δα| ≤ fov / 2.

4.2  Confidence and Detection Probability

Range boost and angle boost

range_boost = max0,  1 − rrange_mangle_boost = max0,  1 − Δαhalf_fov

Confidence

conf = clamp0.15 + 0.83 · range_boost1.35 · 0.55 + 0.45 · range_boost,  0.02,  0.98

A vehicle is missed (skipped) if a uniform random draw exceeds conf.

4.3  Position Noise Model  (Pinhole Pixel-Error)

Effective focal length in pixels

fpx = Wpx2tanfov2

W_px = 1280 pixels  (CAMERA_PIXEL_WIDTH).

Range error coefficient

kr = σpxfpx

σ_px = 1.5 pixels  (CAMERA_SIGMA_PX).  Converts pixel localisation error to metres per metre of range.

Position noise standard deviation

σcam = max0.08,  σ0 + kr · r + kθ · Δα

σ₀ = 0.10 m  (irreducible floor),  k_r from pinhole model,  k_θ = 0.15 m/rad.

Position measurement

xmeas = xtrue + ℔0, σcam2ymeas = ytrue + ℔0, σcam2

| Symbol | Value / Units | Description |
| --- | --- | --- |
| W_px | 1280 px | Image width (CAMERA_PIXEL_WIDTH) |
| σ_px | 1.5 px | Localisation noise in pixels (CAMERA_SIGMA_PX) |
| f_px | px | Effective focal length |
| k_r | m/m | Range noise coefficient = σ_px / f_px |
| k_θ | 0.15 m/rad | Off-axis angle noise coefficient |
| σ₀ | 0.10 m | Irreducible noise floor |
| σ_cam | m | Effective position noise std |
| Δα | rad | Off-axis viewing angle |
| conf | — | Detection confidence ∈ [0.02, 0.98] |

5   DAS Sensor  (Distributed Acoustic Sensing — Fiber Optic)

Optical fiber buried along the road edge. Vehicle weight loads the ground, producing a measurable strain amplitude that decays with perpendicular distance r from the fiber.

5.1  Fiber Geometry

The fiber polyline is the road centreline shifted laterally by fiber_offset_m = 2.0 m. Distance r and fiber arc-length s_fiber are the perpendicular distance and projection of the vehicle onto this polyline.

5.2  DAS Amplitude  (Flamant–Boussinesq Point-Load Model)

Signal amplitude

A = Wr + d02

W = vehicle weight [kg].  d₀ = 0.7 m is a singularity-avoidance floor.

Note: vehicle speed does not appear in the amplitude. Speed information is encoded in the diagonal slope of the space–time (waterfall) plot as r changes over time.

5.3  Trace / Waterfall Generation

Channel index of vehicle

idx = intclampsfiberLfiber · (n − 1),  0,  n − 1

Ideal trace  (Gaussian spreading over ±8 channels)

ideal_trace[idx + k] += exp−k218 · A,     k = −8 … +8

Background noise level

σtrace = σtraffic · noise_std0.02

σ_traffic:  light = 30,  medium = 60,  heavy = 120.

Noisy trace

trace[i] = ideal_trace[i] + ℔0, σtrace2

5.4  SNR and Position Measurement

Signal-to-noise ratio

SNR = Aσtrace

Position uncertainty along fiber  (k_DAS = 2.0 m)

σDAS = max0.8,  kDASSNR

Noisy fiber arc-length measurement

smeas = clampsfiber + ℔0, σDAS2,  0,  Lfiber

5.5  Speed Measurement

Speed noise std

σv = max0.6,  4.0max(1, SNR)

Measured speed (signed along fiber)

vs = sign · max0,  vtrue + ℔0, σv2

sign = +1 if vehicle moves in same direction as fiber tangent, else −1.

Reported speed

speed_mps = vs

Reported heading

heading = θfiber                  if  vs ≥ 0heading = wrap_angleθfiber + π   if  vs < 0

5.6  DAS Confidence

Confidence

confidence = clampSNRSNR + 2,  0.10,  1.00

| Symbol | Value / Units | Description |
| --- | --- | --- |
| W | kg | Vehicle weight (default 1500 kg) |
| r | m | Perpendicular distance from vehicle to fiber |
| d₀ | 0.7 m | Singularity floor (DASSensor.d0_m) |
| A | — | Flamant–Boussinesq amplitude = W / (r + d₀)² |
| n | 1200 | Number of DAS channels (channel_end − channel_start + 1) |
| σ_trace | — | Background noise std of the waterfall trace |
| SNR | — | A / σ_trace |
| σ_DAS | m | Position uncertainty along fiber arc-length |
| σ_v | m/s | Speed measurement noise std |
| k_DAS | 2.0 m | DAS position noise coefficient |
| s_fiber | m | True arc-length of vehicle projection onto fiber |
| s_meas | m | Noisy measured arc-length |

6   Kalman Filter  (Sensor Fusion Layer)

Fuses GPS, camera, and DAS measurements into a single 6-state estimate per vehicle. Position and velocity updates are applied as two sequential updates per tick.

6.1  State Vector

State

ˆx = x  vx  ax  y  vy  ay
              T

6.2  State Transition Matrix F  (constant-acceleration kinematics)

Per-axis 3×3 block

Faxis = 1dt12dt201dt001

Full 6×6 transition matrix

F = block-diagFaxis, Faxis

6.3  Process Noise Covariance Q

Per-axis noise block

Qaxis = σq2 · dt44dt32dt22dt32dt2dtdt22dt1

σ_q = 0.9  (process noise tuning parameter).  Full Q = block-diag(Q_axis, Q_axis).

6.4  Initial State Covariance

P₀

P0 = diagσpos2, σvel2, σacc2, σpos2, σvel2, σacc2

Defaults: σ_pos = 2.5 m,  σ_vel = 2.0 m/s,  σ_acc = 1.0 m/s².

6.5  Predict Step

State prediction

ˆx− = F · ˆx

Covariance prediction

P− = F · P · FT + Q

6.6  Measurement Update

Effective measurement noise is confidence-scaled so that higher-confidence sensors receive proportionally more weight in the fusion.

Effective measurement noise std

σeff = max0.05,  σsensorconfidence

Innovation  (residual)

ν = z − H · ˆx−

Innovation covariance

S = H · P− · HT + R

Kalman gain

K = P− · HT · S−1

State update

ˆx = ˆx− + K · ν

Covariance update  (Joseph form — numerically stable)

P = I − KH · P− · I − KHT + K · R · KT

6.7  Observation Matrices H

Position measurements

Hx = [  1   0   0   0   0   0  ]Hy = [  0   0   0   1   0   0  ]

Velocity measurements

Hvx
                   = [  0   1   0   0   0   0  ]Hvy
                   = [  0   0   0   0   1   0  ]

6.8  Velocity Decomposition from Sensor Readings

Velocity components

vx = speed_mps · cosheading_radvy = speed_mps · sinheading_rad

Velocity noise std — GPS

σvGPS = max0.20,  0.7 + 0.15 · σposconf

Velocity noise std — Camera

σvcam = max0.25,  0.9 + 0.20 · σposconf

Velocity noise std — DAS (fiber)

σvDAS = max1.20,  0.60 · σposconf

6.9  Output Uncertainties

Position uncertainty  (marginalised over x and y)

σpos = 0.5 · P00 + P33

Velocity uncertainty

σvel = 0.5 · P11 + P44

| Symbol | Value / Units | Description |
| --- | --- | --- |
| x̂ | 6×1 | State estimate [x, vx, ax, y, vy, ay]ᵀ |
| P | 6×6 | State error covariance matrix |
| F | 6×6 | State transition matrix |
| Q | 6×6 | Process noise covariance (σ_q = 2.0) |
| H | 2×6 | Observation matrix (position or velocity rows) |
| R | diag | Measurement noise covariance = σ_eff² per channel |
| K | 6×m | Kalman gain matrix |
| ν | m×1 | Innovation (residual) vector |
| σ_eff | m or m/s | Confidence-scaled effective noise std |

6.10  Tuned Noise Parameters  (Stage B v063)

The following parameters were updated based on empirical analysis of simulation results. Original values produced a severely overconfident filter (actual error ~4× the reported σ_pos in 89.6% of steps). The tuned values correct this.

| Symbol | Value / Units | Description |
| --- | --- | --- |
| σ_q | 2.0 (was 0.9) | Process noise — increased to reduce overconfidence |
| σ_p_fiber | 5.0 m (was 2.5 m) | DAS position noise — increased to reduce DAS weight |
| σ_p_camera | 1.3 m | Camera position noise — unchanged |
| σ_p_gps | 1.8 m | GPS position noise — unchanged |

6.11  DAS Velocity Updates  (Diagnostic Step 1)

DAS-derived velocity measurements are currently excluded from the Kalman update step. The DAS speed estimate is obtained by projecting vehicle motion onto the fiber tangent, introducing large noise (σ_v ≈ 4 / max(1, SNR) m/s). Including these updates was found to corrupt the velocity state and degrade position estimates.

Velocity is still estimated via: (1) the constant-acceleration motion model, (2) GPS velocity updates, and (3) camera velocity updates. Only the DAS velocity contribution is suppressed.

7   RMSE Performance Metrics

Computed post-simulation by replaying measurements through the Kalman filter and comparing each estimate to the nearest ground-truth state.

Per-axis RMSE

RMSE
                  x = 1Nix̂i − xtrue,i2RMSE
                  y = 1Niŷi − ytrue,i2

2-D position RMSE

RMSE
              pos = 1Nix̂i − xtrue,i2 + ŷi − ytrue,i2

For individual sensors (GPS, Camera, DAS), the same formula is applied over that sensor's own measurement set  (x_meas, y_meas)  instead of the Kalman estimate.

| Symbol | Value / Units | Description |
| --- | --- | --- |
| N | — | Number of estimates in the evaluation window |
| x̂_i, ŷ_i | m | Kalman filter position estimate at step i |
| x_true,i | m | Ground-truth position at time t_i (nearest-neighbour) |
| RMSE_pos | m | Root-mean-square 2-D position error |

Appendix — Simulation Constants

| Symbol | Value / Units | Description |
| --- | --- | --- |
| DEFAULT_LANE_WIDTH_M | 3.6 m | Default lane width |
| DEFAULT_SPEED_LIMIT_MPS | 13.9 m/s | Default speed limit (~50 km/h) |
| ACCEL_RESPONSE_TAU_S | 1.5 s | First-order speed-tracking τ |
| ACCEL_CMD_DEADBAND_MPS | 0.12 m/s | Acceleration deadband |
| CRUISE_DEADBAND_MPS | 0.18 m/s | Cruise hold deadband |
| DEFAULT_MAX_ACCEL_MPS2 | 1.8 m/s² | Default max acceleration |
| DEFAULT_MAX_DECEL_MPS2 | 2.6 m/s² | Default max deceleration |
| LATERAL_DRIFT_TAU_S | 1.5 s | OU lateral drift τ_lat |
| CAMERA_PIXEL_WIDTH | 1280 px | Reference image width |
| CAMERA_SIGMA_PX | 1.5 px | Camera localisation noise |
| fiber_offset_m | 2.0 m | Fiber lateral offset from road centreline |
| d0_m | 0.7 m | Flamant–Boussinesq singularity floor |
| k_DAS | 2.0 m | DAS position noise coefficient |
| Kalman σ_q | 0.9 | Process noise parameter |
| Kalman σ_p_camera | 1.3 m | Default camera position noise in Kalman R |
| Kalman σ_p_gps | 1.8 m | Default GPS position noise in Kalman R |
| Kalman σ_p_fiber | 2.5 m | Default DAS position noise in Kalman R |