TRACKING AUDIT REPORT

Enhanced Diagnostic Analysis

Simulation: /sessions/nifty-dreamy-turing/mnt/final proj/outputs/_sample_audit_report

Generated: 2026-05-11  |  1 Tracks  |  3 Sensor Types

# 1. Executive Summary

Overall tracking quality is **medium** — RMSE 0.49 m with 45% prediction-only rows; the filter coasts noticeably but stays near truth. The biggest error source is *GPS measurement noise / bias*. Most suspicious behavior: recurring 'sensor_dropout' anomalies (3 occurrences). Best-performing sensor: Camera; weakest: GPS. Weakest time window: t = 3.0–4.0 s (mean err 0.72 m). The system relied heavily on prediction-only Kalman rows (45% of all rows). GPS measurements appear unreliable — see the sensor-quality section.

| Rank | Finding | Impact | Status |
| --- | --- | --- | --- |
| #1 | Overall quality: **medium** — RMSE 0.49 m with 45% prediction-only rows; the filter coasts noticeably but stays near truth | Low | ✓ OK |
| #2 | Biggest error source: GPS measurement noise / bias | Medium | ⚠ Warning |
| #3 | Most suspicious behavior: recurring 'sensor_dropout' anomalies (3 occurrences) | Medium | ⚠ Warning |
| #4 | Best sensor: Camera | Low | ✓ OK |
| #5 | Weakest sensor: GPS | Low | ✓ OK |
| #6 | Weakest time window: t = 3.0–4.0 s (mean err 0.72 m) | Low | ✓ OK |
| #7 | Relied on prediction-only rows: yes (45% of all rows) | Low | ✓ OK |
| #8 | Too few sensor measurements: no | Low | ✓ OK |
| #9 | Suspicious noisy sensor: GPS | Medium | ⚠ Warning |
| #10 | Vehicle fragmentation suspected: no | Low | ✓ OK |

# 2. Sensor Coverage Analysis

Across 1 track(s) the audit recorded 13 GPS, 37 camera and 93 DAS measurements plus 107 prediction-only rows (44.6% of all rows). 3 time window(s) had only one sensor type active (see fragmentation / FOV sections for the likely cause).

| Track | GPS | Cam | DAS | Pred | Rows | %pred | Longest meas-gap (s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| T000001 | 13 | 37 | 93 | 107 | 240 | 44.6 | 0.60 |

3 time window(s) had only one sensor type active (see fragmentation / FOV sections for the likely cause).

ℹ Sensor coverage looks sufficient What happened: GPS=13, Cam=37, DAS=93, prediction-only=44.6%, longest gap=0.60 s. What it means: Track T000001 received a balanced mix of measurements with no large gap. The Kalman has enough evidence to stay close to truth. Why it matters: Healthy coverage is the baseline for trustworthy tracking.

Evidence:

GPS = 13 measurements, Camera = 37, DAS = 93.

Prediction-only rows = 44.6% of the track.

Longest gap between real measurements = 0.60 s (track duration = 11.95 s).

# 3. Sensor Quality / Noise Analysis

Per-sensor noise / bias is computed by comparing each accepted measurement against the oracle ground truth at the same timestep. ‘σ vs err’ flags whether the reported sigma matches reality — a ratio far from 1.0 means the sensor model is mis-calibrated.

| Sensor | N | Mean err (m) | Max err (m) | RMSE (m) | Bias x (m) | Bias y (m) | Mean sigma | Sigma vs err |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | 13 | 3.73 | 7.48 | 4.07 | 0.41 | -1.22 | 2.80 | σ ≈ actual |
| Camera | 37 | 0.39 | 0.83 | 0.44 | 0.00 | 0.03 | 0.30 | σ ≈ actual |
| DAS | 93 | 0.61 | 1.54 | 0.70 | -0.06 | 0.06 | 0.44 | σ ≈ actual |

ℹ GPS behaves as expected What happened: GPS: mean err 3.73, σ 2.80, bias (0.41, -1.22). What it means: GPS mean error and bias are consistent with the reported σ — no immediate noise / calibration concern. Why it matters: This sensor is likely a reliable contributor to the fusion.

Evidence:

13 GPS measurements compared to ground truth.

Mean error = 3.73, max error = 7.48, RMSE = 4.07.

Bias = (Δx=0.41, Δy=-1.22).

Mean reported σ = 2.80.

ℹ Camera behaves as expected What happened: Camera: mean err 0.39, σ 0.30, bias (0.00, 0.03). What it means: Camera mean error and bias are consistent with the reported σ — no immediate noise / calibration concern. Why it matters: This sensor is likely a reliable contributor to the fusion.

Evidence:

37 Camera measurements compared to ground truth.

Mean error = 0.39, max error = 0.83, RMSE = 0.44.

Bias = (Δx=0.00, Δy=0.03).

Mean reported σ = 0.30.

ℹ DAS behaves as expected What happened: DAS: mean err 0.61, σ 0.44, bias (-0.06, 0.06). What it means: DAS mean error and bias are consistent with the reported σ — no immediate noise / calibration concern. Why it matters: This sensor is likely a reliable contributor to the fusion.

Evidence:

93 DAS measurements compared to ground truth.

Mean error = 0.61, max error = 1.54, RMSE = 0.70.

Bias = (Δx=-0.06, Δy=0.06).

Mean reported σ = 0.44.

# 4. Error Spike Investigation

Every detected spike or jump is paired with the local sensor context (what was available before / during / after) and a rule-based explanation that distinguishes confirmed facts from likely causes.

No major error spikes were detected in any track.

# 5. Track Fragmentation Analysis

1 oracle vehicle(s) were observed across 1 track ID(s).  Each oracle vehicle maps to a single track — no fragmentation suspected.

| Vehicle | # tracks | Tracks | Filtered tracks |
| --- | --- | --- | --- |
| veh1 | 1 | T000001 [0.05-12.00s, 240 rows] | — |

# 6. Segment Transition Analysis

Compares the segment transitions the report would emit against the transitions implied by oracle lane_id over time.  Mismatches usually indicate a coverage / drift issue rather than a transition-logic bug.

| Track | Vehicle | Reported transitions | Oracle transitions | Match |
| --- | --- | --- | --- | --- |
| T000001 | veh1 | seg1→seg2@7.40s | seg1 → seg2 | yes |

ℹ Segment transition logic appears consistent What happened: Reported transitions match the oracle for every meaningful track. What it means: The segment-assignment pipeline produced the same story as the ground truth. Why it matters: Trustworthy transitions are necessary for any per-segment analysis.

Evidence:

Per-track row in the table above shows match=yes for every row.

# 7. Camera Geometry / FOV Analysis

Per-track camera measurement counts, time windows of camera presence, and a sanity check on the camera σ vs actual error.  Conclusions about FOV are heuristic — the audit data does not carry the camera polygon itself.

| Track | Cam meas | Cam window (s) | Pre-tr / post-tr | Mean σ | Mean err |
| --- | --- | --- | --- | --- | --- |
| T000001 | 37 | 10.35 | 30/7 | 0.30 | 0.39 |

ℹ Camera contributes mostly before the transition What happened: Camera meas pre/post first transition: 30/7. What it means: The camera dominates the early segment and then largely disappears; the post-transition tracking depends on DAS. Why it matters: Asymmetric camera coverage explains why error often rises right after a segment transition even when the filter is healthy.

Evidence:

Camera-window length: 10.35s with the bulk before the transition.

Possible causes:

FOV layout intentionally favors the early segment.

Next checks:

Decide whether post-segment camera coverage is worth adding.

# 8. DAS Quality Analysis

DAS measurement counts, SNR distribution, and the Kalman error in DAS-only windows.  Where SNR data is available, correlations with tracking error are flagged as evidence of σ-from-SNR mis-calibration.

| Track | DAS meas | Mean SNR | Min SNR | Low-SNR rows | Mean err |
| --- | --- | --- | --- | --- | --- |
| T000001 | 93 | 23.85 | 9.26 | 0 | 0.34 |

# 9. Kalman Behavior Analysis

For each track the Kalman is evaluated on: how often it received real updates (vs prediction), whether σ grows in gaps and shrinks after measurements, and whether the reported σ matches the actual error.  Strong mismatches indicate a tuning issue — not a bug.

| Track | % pred-only | σ growth in gaps | σ shrink after meas | σ vs actual err |
| --- | --- | --- | --- | --- |
| T000001 | 44.6 | 33/35 | 31/35 | σ ≈ err (healthy) |

ℹ Kalman behaves correctly What happened: σ tracks actual error, σ grows during gaps, shrinks after measurements. What it means: All meaningful tracks show the textbook Kalman pattern: uncertainty rises while coasting and falls when sensor data returns, and the reported uncertainty matches the actual error well. Why it matters: No tuning intervention needed for the filter itself.

Evidence:

1/1 tracks classified healthy.

# 10. Recommendations & Next Investigations

Concrete checks aggregated from the diagnostic sections.  Items are ordered by severity (critical → warning → info) and de-duplicated.  Each is linked to the interpretation that suggested it.

(Camera contributes mostly before the transition) Decide whether post-segment camera coverage is worth adding.

# 11. Track Overview

1 track(s) recorded.  Global RMSE: 0.49 m.  Global max error: 1.52 m.  Prediction-only rows: 107.

| Track | Vehicle | Duration (s) | RMSE (m) | Max err (m) | GPS | Camera | DAS | Pred-only | % Pred | Segments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T000001 | veh1 | 11.95 | 0.40 | 1.32 | 13 | 37 | 93 | 107 | 44% | seg1 → seg2 |

## Segment Transitions

1 transition(s) across all tracks.

| Track | Time (s) | From segment | To segment |
| --- | --- | --- | --- |
| T000001 | 7.40 | seg1 | seg2 |

# 12. Sensor σ Calibration Check

Compares each sensor's declared measurement noise (sigma_m) against actual error from ground truth.  A well-calibrated sensor has mean actual error ≈ mean declared σ.  If actual error > 1.2×σ the filter over-trusts that sensor and the R-matrix entry should be increased.

| Sensor | N meas. | Mean actual err (m) | Max actual err (m) | RMSE (m) | Mean declared σ (m) | Actual / σ ratio | % over-trusted | Verdict |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | 13 | 3.730 | 7.479 | 4.068 | 2.798 | 1.33 | 53.8% | ⚠ Over-trusted |
| Camera | 37 | 0.390 | 0.831 | 0.440 | 0.303 | 1.29 | 48.6% | ⚠ Over-trusted |
| DAS | 93 | 0.615 | 1.543 | 0.704 | 0.442 | 1.39 | 52.7% | ⚠ Over-trusted |

⚠ CALIBRATION WARNING GPS: actual error is 1.33× declared σ (mean err 3.73 m vs σ 2.80 m). Increase R_gps in the Kalman noise matrix.

⚠ CALIBRATION WARNING Camera: actual error is 1.29× declared σ (mean err 0.39 m vs σ 0.30 m). Increase R_camera in the Kalman noise matrix.

⚠ CALIBRATION WARNING DAS: actual error is 1.39× declared σ (mean err 0.61 m vs σ 0.44 m). Increase R_das in the Kalman noise matrix.

# 13. Per-Axis Error Breakdown (X vs Y)

Scalar RMSE hides directional bias.  A dominant Y error (lateral) usually points to insufficient lateral process noise (Q_yy too small) or missing sensors that constrain cross-track position.  A dominant X error suggests speed-model mismatch.

## Track T000001 — veh1

| Axis | Mean ¦error¦ (m) | RMSE (m) | Signed bias (m) | Dominant? |
| --- | --- | --- | --- | --- |
| X (longitudinal) | 0.185 | 0.231 | -0.003 | — |
| Y (lateral) | 0.309 | 0.435 | +0.045 | Yes |

⚠ Y (lateral) error dominates by 1.7×. Consider increasing Q_yy (lateral process noise) so the filter tracks lateral motion more responsively.

# 14. Sensor Coverage Timeline

Each row = one second of simulation.  ✓ = at least one accepted measurement;  — = sensor silent (filter coasted).  Seconds where ALL sensors are silent are the primary source of position drift.

## Track T000001 — veh1

| Second (s) | GPS | Camera | DAS | Status |
| --- | --- | --- | --- | --- |
| 0–1 | — | ✓ | — | ✓ sensor active |
| 1–2 | — | ✓ | — | ✓ sensor active |
| 2–3 | — | ✓ | — | ✓ sensor active |
| 3–4 | — | ✓ | — | ✓ sensor active |
| 4–5 | ✓ | ✓ | — | ✓ sensor active |
| 5–6 | ✓ | ✓ | — | ✓ sensor active |
| 6–7 | ✓ | ✓ | — | ✓ sensor active |
| 7–8 | ✓ | ✓ | ✓ | ✓ sensor active |
| 8–9 | — | ✓ | ✓ | ✓ sensor active |
| 9–10 | — | ✓ | ✓ | ✓ sensor active |
| 10–11 | — | ✓ | ✓ | ✓ sensor active |
| 11–12 | — | — | ✓ | ✓ sensor active |
| 12–13 | — | — | ✓ | ✓ sensor active |
| 13–14 | — | — | — | ✗ prediction only |

Dead seconds (all sensors silent): 1 / 14 (7%)

# 15. Cold-Start Latency

Time from the first trajectory row to the first moment the position error drops below 0.5 m.  A long cold-start indicates the filter is initialized far from the true position or the initial covariance P₀ is too small.

| Track | Vehicle | Track start (s) | First error < 0.5 m at (s) | Cold-start latency (s) | Initial error (m) |
| --- | --- | --- | --- | --- | --- |
| T000001 | veh1 | 0.050 | 0.050 | 0.000 s | 0.152 |

# 16. Speed vs Position Error

Position error bucketed by estimated vehicle speed.  If error grows with speed, the process noise Q or motion model is misspecified for high-speed dynamics.  If error is flat, sensor dropout is the dominant driver.

| Speed bucket | N rows | Mean error (m) | Max error (m) | Trend vs prev |
| --- | --- | --- | --- | --- |
| 0–5 m/s | 0 | — | — | — |
| 5–10 m/s | 0 | — | — | — |
| 10–15 m/s | 41 | 0.360 | 1.316 | — |
| 15–20 m/s | 199 | 0.408 | 1.523 | ↑ growing |
| 20+ m/s | 0 | — | — | — |

✓ Error does not strongly correlate with speed — sensor dropout is the primary error driver, not model mismatch.