# SimStudio — Anomaly Scenarios

Catalogue of the 8 anomaly classes the simulator supports and the 5 scenarios authored for each class.

# Overview

This document catalogues the anomaly classes that the SimStudio traffic simulator can reproduce and the scenarios authored for each class. All scenarios live under outputs/test_scenarios/anomalies/<class>/ and each scene carries one DAS fibre, one GPS receiver and one camera looking along the road — so every sensor modality is represented in every run. The five scenarios per class deliberately vary road length, road curvature, speed limit, vehicle count and vehicle mass so the raw sensor traces look different run-to-run, while the anomaly itself is preserved.

All scenarios were verified headlessly through scripts/make_anomaly_scenarios.py — 40/40 currently pass their class-specific physical invariant (e.g. the braker reaches a near-stop, the tailgater’s time-headway is below 0.7 s, the stalled vehicle does not drift, the collision emits a world.collision event, etc.).

# Standard Sensor Layout (all scenarios)

Every scenario is built on the same minimal scaffold so the sensor channels are comparable across classes:

One straight or gently-curved one-way segment.

One DAS sensor running along the full length of the segment (fibre offset 2.0 m, d0 0.7 m, 30 Hz).

One GPS receiver at the road midpoint, σ 2.5 m, 5 Hz.

One camera positioned just before the segment start, 70° FOV, 10 Hz.

auto_spawn disabled — vehicle list is whatever the scenario authors.

# Sudden Braking

Definition. A vehicle applies an emergency deceleration well below the normal IDM comfort limit (roughly -7 to -9 m/s²) and drops to a near-stop within a few seconds. In this simulator the behaviour is injected through the per-vehicle overrides a_cmd_override_mps2 and a_cmd_override_until_t, which bypass IDM for a bounded time window.

Sensor signatures. DAS sees a short, sharp negative strain-rate transient centred on the braking vehicle; Camera reports a rapid drop in tracked speed; GPS shows a step change in the derived velocity.

Scenarios (5):

| # | Name | Vehicles | Road | Limit | Description / timing |
| --- | --- | --- | --- | --- | --- |
| 1 | 01_lone_braker | 1 | 300 m | 50 km/h | A single car at 50 km/h begins braking at -7 m/s² for 3 s starting at t=0 and stops by ~t=2 s; IDM releases afterwards and the car re-accelerates. |
| 2 | 02_cascade_brake | 2 | 400 m | 50 km/h | A leader at s=80 m brakes at -6 m/s² for 4 s; a follower at s=20 m uses default IDM and must react to the braking leader. Classic cascade profile. |
| 3 | 03_curve_braker | 1 | 500 m (curved) | 50 km/h | A lone car brakes at -5.5 m/s² for 5 s while travelling along a gentle arc; tests GPS+camera geometry on a curved trajectory. |
| 4 | 04_high_speed | 1 | 600 m | 80 km/h | An 80 km/h car panic-brakes at -8 m/s² for 3.5 s; DAS sees a higher-amplitude, shorter-duration signature than the 50 km/h case. |
| 5 | 05_truck | 1 | 500 m | 50 km/h | A 12-ton truck brakes at -5 m/s² for 4 s; DAS strain amplitude scales roughly with vehicle weight, producing a much larger peak than a passenger car. |

# Tailgating

Definition. A follower maintains a time-headway substantially below the normal ~1.5 s (IDM default T). The simulator enforces this through per-vehicle overrides min_gap_m (IDM s₀) and time_headway_s (IDM T). A follower with s₀≈0.5 m and T≈0.3 s is a classical tailgate driver.

Sensor signatures. DAS shows two peaks whose separation is much smaller than the normal convoy spacing; Camera sees two objects persistently within a short longitudinal distance; GPS tracks confirm that the pair share nearly the same velocity vector.

Scenarios (5):

| # | Name | Vehicles | Road | Limit | Description / timing |
| --- | --- | --- | --- | --- | --- |
| 1 | 01_basic_pair | 2 | 400 m | 50 km/h | Slow leader at 8 m/s and tailgating follower (s₀=0.5 m, T=0.3 s) starting 60 m behind at 13.9 m/s; settles to gap ≈3 m. |
| 2 | 02_chain_of_three | 3 | 500 m | 50 km/h | Slow leader at 8 m/s, a tight middle follower (s₀=0.5 m, T=0.3 s), and a normal third car. The middle car clearly tailgates while the rear car keeps a comfortable gap. |
| 3 | 03_behind_truck | 2 | 500 m | 50 km/h | Aggressive follower (s₀=0.7 m, T=0.25 s) tailgates a 12-ton truck at 9 m/s; shows heavy-leader + light-follower DAS asymmetry. |
| 4 | 04_curve_pair | 2 | 500 m (curved) | 50 km/h | Same classic tailgate pair but on a gently curved road so the two DAS peaks trace a curved line along the fibre channels. |
| 5 | 05_high_speed | 2 | 900 m | 80 km/h | High-speed tailgate: leader at 16.7 m/s and follower at 22.2 m/s with T=0.4 s. Equilibrium gap is ~8 m but time-headway is ~0.4 s — still clearly tailgating. |

# Stalled Vehicle

Definition. A vehicle that has come to a stop in-lane and cannot move — e.g. breakdown, mechanical failure. In the simulator this is expressed with frozen=True and an envelope pinned to v=0, which also pins its lateral offset so it does not drift. Other vehicles treat the stall as a zero-velocity leader through IDM.

Sensor signatures. DAS sees a low-rate quasi-static strain signature that does not translate along the fibre; Camera reports a tracked object with zero velocity for the entire run; GPS returns a constant position.

Scenarios (5):

| # | Name | Vehicles | Road | Limit | Description / timing |
| --- | --- | --- | --- | --- | --- |
| 1 | 01_single_stall | 2 | 300 m | 50 km/h | One stalled car at s=150 m; an approaching car from s=10 m at 13.9 m/s brakes naturally under IDM and settles to a stop behind the stall. |
| 2 | 02_three_car_jam | 4 | 500 m | 50 km/h | Stalled car at s=200 m; three additional cars arrive from behind and form a natural queue — multi-peak stationary convoy on DAS. |
| 3 | 03_stalled_truck | 2 | 500 m | 50 km/h | A 12-ton stalled truck at s=180 m; DAS amplitude is ~8x a passenger car, but no temporal dynamics. |
| 4 | 04_curve_stall | 2 | 500 m (curved) | 50 km/h | A stalled car at s=250 m on a curved segment; an approaching vehicle rounds the bend and brakes to a stop behind it. |
| 5 | 05_double_stall | 3 | 500 m | 50 km/h | Two cars stalled back-to-back at s=180 and s=220 m; an approaching car stops behind the pair. Rare but informative double-peak stationary signature. |

# Obstacle in Lane

Definition. A non-vehicle stationary object blocking a lane — pedestrian, debris, animal, or barrier. Represented with the same frozen mechanism as a stalled vehicle but tagged with object_type so the GUI renders a different glyph. Unlike a stalled car, the object's mass is typically much smaller (5–200 kg), which changes the DAS amplitude significantly.

Sensor signatures. DAS signature amplitude scales with mass (a 5 kg cone is essentially DAS-invisible; a 200 kg debris heap is detectable); Camera is the primary sensor — object_type gives class information; GPS is blind to non-vehicle obstacles.

Scenarios (5):

| # | Name | Vehicles | Road | Limit | Description / timing |
| --- | --- | --- | --- | --- | --- |
| 1 | 01_pedestrian | 2 | 300 m | 50 km/h | 80 kg pedestrian at s=150 m; an approaching car at 13.9 m/s brakes under IDM. |
| 2 | 02_debris | 2 | 300 m | 50 km/h | 200 kg debris (fallen cargo) at s=140 m; approaching car brakes under IDM. |
| 3 | 03_animal | 2 | 400 m | 50 km/h | 60 kg deer frozen in the lane at s=170 m; approaching car brakes under IDM. |
| 4 | 04_barrier | 2 | 400 m | 50 km/h | 5 kg traffic cone at s=180 m — camera-visible, DAS-invisible. An approaching car still brakes under IDM because the cone is treated as a leader at v=0. |
| 5 | 05_two_pedestrians | 3 | 500 m | 50 km/h | Two pedestrians mid-block at s=190 and s=198 m; approaching car brakes for the nearest. |

# Weaving

Definition. Sustained sinusoidal lateral oscillation — the classic drunk-/distracted-driver motion. Expressed in the simulator through lateral_mode="weave" plus lateral_weave_amplitude_m and lateral_weave_period_s. The vehicle’s longitudinal dynamics remain normal; only the lane-lateral offset oscillates.

Sensor signatures. DAS sees a lateral-offset-modulated amplitude pattern at the weaver’s longitudinal position; Camera tracks the lane-lateral position drifting back and forth across the lane centreline; GPS lateral noise is too large to resolve a ~2 m weave directly.

Scenarios (5):

| # | Name | Vehicles | Road | Limit | Description / timing |
| --- | --- | --- | --- | --- | --- |
| 1 | 01_solo_weave | 1 | 400 m | 50 km/h | Single car, amp=2.5 m, period=4 s. Two full oscillation cycles in an 8 s run. |
| 2 | 02_weaver_in_traffic | 3 | 500 m | 50 km/h | One weaver (amp=2.2 m, period=5 s) mixed with two normal cars; tests whether the weaver is separable from normal OU lateral drift. |
| 3 | 03_truck_weave | 1 | 500 m | 50 km/h | 12-ton truck slow-weaving with amp=3.0 m, period=6 s — the large lateral amplitude coupled with heavy mass is a very strong DAS signature. |
| 4 | 04_curve_weave | 1 | 500 m (curved) | 50 km/h | Weaving (amp=2.3 m, period=4.5 s) superimposed on a curved road: lateral offset is w.r.t. the local road tangent. |
| 5 | 05_high_speed_weave | 1 | 700 m | 80 km/h | High-speed weaver at 22.2 m/s with a fast 3 s period — higher longitudinal resolution on DAS, more demanding lane-keeping. |

# Lane Straddling

Definition. A vehicle held on or near one lane edge for a sustained interval rather than returning to the lane centreline. Expressed with lateral_mode="straddle" and lateral_fixed_offset_m. Unlike weaving, the offset is static — the vehicle is riding the lane marking.

Sensor signatures. DAS sees a shifted transverse strain signature (stronger on one side of the fibre than normal); Camera reports a persistent lane-lateral offset; GPS cannot resolve the sub-lane offset but the fused track shows a lane-edge bias.

Scenarios (5):

| # | Name | Vehicles | Road | Limit | Description / timing |
| --- | --- | --- | --- | --- | --- |
| 1 | 01_left_edge | 1 | 400 m | 50 km/h | Single car pinned to +1.8 m (left edge of a 3.6 m lane) for the entire run. |
| 2 | 02_right_edge | 1 | 400 m | 50 km/h | Mirror case: single car pinned to -1.8 m (right edge) for the entire run. |
| 3 | 03_truck_straddle | 1 | 500 m | 50 km/h | 12-ton truck pinned to +1.8 m — high-severity DAS signature and clear camera offset. |
| 4 | 04_pair_with_straddler | 2 | 500 m | 50 km/h | Normal leader on the centreline followed by a straddler at +1.8 m; checks whether the sensor fusion can separate the two lateral profiles. |
| 5 | 05_curve_straddle | 1 | 500 m (curved) | 50 km/h | Pinned lateral offset on a curved segment — the offset is applied along the local lane-perpendicular direction. |

# Collision

Definition. Two vehicles occupy the same longitudinal position, i.e. the inter-vehicle gap reaches zero. Expressed through per-vehicle overrides allow_collision=True and disable_idm=True on the follower, which bypass the numerical guards that would normally prevent overlap. The simulator emits a world.collision event at impact.

Sensor signatures. DAS records a sharp impulse at the impact site followed by a quasi-static signature as the wreckage remains in place; Camera reports two tracks converging to a single position; GPS shows both tracks going to zero velocity simultaneously.

Scenarios (5):

| # | Name | Vehicles | Road | Limit | Description / timing |
| --- | --- | --- | --- | --- | --- |
| 1 | 01_rear_end | 2 | 300 m | 50 km/h | Follower at 13.9 m/s with IDM disabled charges into a frozen leader 70 m ahead; impact around t=5 s. |
| 2 | 02_high_speed_rear_end | 2 | 500 m | 80 km/h | Same profile at 22.2 m/s; impact is earlier and the DAS impulse sharper. |
| 3 | 03_truck_into_car | 2 | 500 m | 50 km/h | Heavy truck follower collides with a passenger car; asymmetric mass signature. |
| 4 | 04_car_into_truck | 2 | 500 m | 50 km/h | Passenger car collides with a stopped 12-ton truck; the fixed-mass target dominates DAS. |
| 5 | 05_chain_collision | 3 | 500 m | 50 km/h | Stopped leader + colliding middle car + trailing normal car that brakes naturally behind the wreck. |

# Speeding

Definition. Vehicle speed exceeds the posted segment limit by a substantial margin. Expressed with ignore_speed_limit=True and an envelope (speed_min/max, target_speed) tuned above the limit. IDM still runs, so a speeder stuck behind slow traffic will be capped by its leader.

Sensor signatures. DAS sees a longer, faster-moving peak with above-normal amplitude; Camera reports a speed above the segment limit; GPS confirms the elevated velocity. Critically — all three sensors must agree the speed is elevated, otherwise the detector should suspect a Camera or GPS glitch.

Scenarios (5):

| # | Name | Vehicles | Road | Limit | Description / timing |
| --- | --- | --- | --- | --- | --- |
| 1 | 01_solo_speeder | 1 | 500 m | 50 km/h | Solo car with ignore_speed_limit on; target 30 m/s (≈108 km/h) against a 50 km/h limit. |
| 2 | 02_speeder_in_traffic | 3 | 600 m | 50 km/h | Speeder placed ahead of two law-abiding cars (target 28 m/s) — free to accelerate without being capped by a slow leader. |
| 3 | 03_truck_speeding | 1 | 600 m | 50 km/h | Speeding 12-ton truck — rarer in practice and combines elevated DAS amplitude with elevated speed. |
| 4 | 04_long_straight | 1 | 1000 m | 50 km/h | Sustained speeder on a 1 km straight segment; long DAS signature well suited to training a long-window detector. |
| 5 | 05_curve_speeder | 1 | 500 m (curved) | 50 km/h | Speeder on a gently curved segment; stresses the heading-based DAS channel mapping. |