TRACKING AUDIT REPORT

Enhanced Diagnostic Analysis

Simulation: simstudio_export_20260510_203004

Generated: 2026-05-10  |  2 Tracks  |  3 Sensor Types

# 1. Executive Summary

This enhanced audit examines two tracks (T000001 and T000002) produced by a 3-sensor fusion system (GPS + Camera + DAS) with a Kalman filter. The dominant problem is not filter malfunction — the Kalman behaves correctly — but sensor scarcity: 78% of all state-estimate rows are prediction-only, meaning the filter is coasting on its motion model without real corrections. The key root causes, ranked by impact, are below.

| Rank | Finding | Impact | Status |
| --- | --- | --- | --- |
| #1 | GPS noise model underestimates actual error (sigma 2.78 m vs actual 3.63 m avg, up to 8.1 m max) | Filter over-trusts GPS → position bias | ⚠ Critical |
| #2 | 78% of Kalman rows are prediction-only — sensors too sparse to control drift | Unchecked drift, error up to 1.84 m | ⚠ Critical |
| #3 | GPS completely absent for T000002 — one track has zero GPS corrections | No global position anchor on T000002 | ⚠ High |
| #4 | Camera FOV is front-weighted: 37 measurements before seg transition, only 15 after | Post-transition tracking quality drops | ⚠ High |
| #5 | Segment assignment inconsistency: T000001 shows seg1→seg2 at t=7.39s; oracle is always seg2 | Wrong lane reporting for ~half the track | ⚠ Medium |
| #6 | 191 DAS measurements skipped (vehicle/track ended) — DAS coverage extends past vehicle lifetime | Wasted processing, potential ghost detections | ℹ Low |

# 2. Sensor-by-Sensor Performance

Each sensor is evaluated on three dimensions: (a) measurement accuracy relative to ground truth, (b) calibration — whether the declared sigma matches the actual error, and (c) coverage — how many corrections it delivered and at what rate.

| Sensor | Mean Err (m) | Max Err (m) | RMSE (m) | Mean σ declared (m) | Bias X / Y (m) | Calibration |
| --- | --- | --- | --- | --- | --- | --- |
| GPS | 3.63 | 8.09 | 3.99 | 2.78 ← UNDERESTIMATES | +0.12 / +0.06 | ⚠ Over-trusted |
| Camera | 0.45 | 1.33 | 0.52 | 0.36 ✓ matches | +0.06 / +0.07 | ✓ Well-calibrated |
| DAS | 0.53 | 1.67 | 0.62 | 0.36 ✓ matches | +0.03 / +0.01 | ✓ Well-calibrated |

## 2.1  GPS — The Problematic Sensor

GPS is the only sensor with a sigma miscalibration. The filter assigns it σ = 2.78 m on average, but the actual measured error is 3.63 m on average (max 8.09 m). This means the Kalman filter trusts GPS more than it should. When a GPS measurement arrives, it pulls the state estimate toward the (noisy) GPS reading with more force than is warranted, introducing a position bias that subsequent predictions propagate forward.

WHY THIS MATTERS A Kalman filter weighs sensor updates by the inverse of their noise covariance (R matrix). If R is set too low for GPS, the filter over-weights each GPS fix. The result is not catastrophic here (only 15 GPS measurements total), but if GPS rate increases, this miscalibration will amplify positional error, not reduce it.

- Actual GPS error exceeds declared sigma in every recorded measurement.

- The bias is consistent: +0.12 m in X and +0.06 m in Y — a systematic offset, not pure noise.

- T000002 has zero GPS measurements at all, suggesting the GPS antenna / zone does not cover segment 2.

## 2.2  Camera — Best Sensor, Too Sparse

Camera is well-calibrated (σ matches actual error) and the most accurate sensor by a wide margin (RMSE 0.52 m). Its problem is coverage: it contributes only 52 measurements over a 13.78-second track (≈ 4 Hz effective rate), and it is heavily front-loaded — 37 measurements before the segment transition at t=7.39 s and only 15 after. The camera essentially disappears in the second half of the track.

- Camera interval: ~0.25 s mean, but with gaps up to 1.06 s.

- T000002 gets only 8 camera measurements over 6.38 s — nearly invisible.

- Error during camera-active periods: mean 0.43 m. Error during prediction-only periods immediately after camera dropout: rises sharply.

## 2.3  DAS — High Rate but Limited Coverage Zone

DAS is the most frequent sensor at ≈25 Hz (0.04 s intervals) and is well-calibrated. It accounts for the majority of real corrections (328 accepted out of 519 generated). The 191 skipped measurements all carry reason 'vehicle/track ended' — meaning the DAS fiber kept generating detections after the vehicle had left its zone. This is harmless but suggests the DAS end-of-track gate could be tightened. Crucially, DAS provides no global position — it measures along-fiber position, so it cannot anchor the X/Y estimate without camera or GPS co-measurement.

# 3. Coverage Analysis — Why 78% is Prediction-Only

| Track | GPS meas. | Cam. meas. | DAS meas. | Pred-only rows | % Pred-only | Longest gap (s) |
| --- | --- | --- | --- | --- | --- | --- |
| T000001 (veh1, 13.78 s) | 15 (1.3%) | 52 (4.4%) | 166 (13.9%) | 964 | 80.9% | 0.43 s |
| T000002 (veh2, 6.38 s) | 0 (0%) | 8 (1.3%) | 162 (26.9%) | 435 | 72.3% | 0.04 s |

T000001 spends 80.9% of its 1,191 Kalman rows in prediction-only mode. The longest prediction-only block is 0.43 s (t = 0.87–1.28 s, 32 consecutive rows). During this block the position error climbs toward its simulation peak of 1.84 m. All 10 worst-error moments occur inside prediction-only stretches.

## 3.1  Root Cause: Update Rate Gap

The sensor update rates, compared to the Kalman step rate (≈130 Hz = 0.0077 s/step), are:

| Sensor | Mean interval (s) | Effective rate (Hz) | Steps/update | Coverage |
| --- | --- | --- | --- | --- |
| GPS | 0.21 s | ~5 Hz | ~27 steps | T000001 only, partial |
| Camera | 0.25 s mean / 1.06 s max | ~4 Hz (nominal) | ~32 steps | Front-weighted, FOV gap after seg transition |
| DAS | 0.04 s | ~25 Hz | ~5 steps | Best coverage, no X/Y anchor |
| Kalman step | ~0.0077 s | ~130 Hz | 1 step | Every step |

Between any two DAS updates (≈5 Kalman steps) the filter predicts without correction. Between two GPS or camera updates (≈27–32 steps) the filter coasts for a substantial window. The Kalman filter itself is healthy — sigma grows during gaps and shrinks correctly after measurements (confirmed: 216/217 growth events, 217/217 shrink events). The problem is purely that real measurements are rare.

# 4. Error Spike Analysis

The worst position errors cluster around specific time windows. The following table maps each peak-error moment to its sensor context and probable cause.

| Track | Time (s) | Error (m) | Update kind | Sigma (m) | Probable cause |
| --- | --- | --- | --- | --- | --- |
| T000001 | t ≈ 4.50–4.64 | 1.68–1.84 m ← PEAK | Prediction-only | 0.78–0.93 m | Camera dropout gap (0.51 s, t=4.13–4.65 s): 32 consecutive prediction steps. Velocity model drifts in Y direction. |
| T000001 | t ≈ 3.55–3.60 | 1.45 m | Prediction-only | 0.50 m | Camera dropout (0.30 s); estimated Y-position lags true Y by ~1.5 m. |
| T000002 | t ≈ 0.00–0.69 | 0.87 m (avg), 1.83 m (peak) | Prediction-only (72%) | 0.45–0.90 m | Track cold-start: no camera, only DAS (low SNR 15.3 mean). Filter initialized far from truth; requires 0.7 s to converge. |
| T000002 | t ≈ 0.87–3.59 | Camera dropout (0.82 s max gap) | Mostly prediction-only | Growing | Camera disappears for 2.72 s — DAS-only cannot constrain lateral position. |

KEY INSIGHT — Y-axis drift pattern All peak errors share the same signature: the estimated Y position lags the true Y position. X estimates stay relatively close. This indicates the Kalman process noise Q is too low for lateral (Y-axis) dynamics — the filter is too confident in its straight-line prediction and resists lateral corrections. Increasing Q_yy (lateral process noise) will improve Y tracking without requiring more sensors.

# 5. Kalman Filter Health

The Kalman filter is functioning correctly in terms of uncertainty propagation. This section shows the evidence, plus two tuning opportunities.

| Metric | T000001 | T000002 | Healthy? | Evidence |
| --- | --- | --- | --- | --- |
| σ grows during prediction-only gaps | 216 / 217 gaps | 160 / 161 gaps | ✓ Yes | Textbook behavior |
| σ shrinks after measurement update | 217 / 217 | 161 / 161 | ✓ Yes | Kalman gain working |
| σ tracks actual error magnitude (not over-optimistic) | σ ≈ err (healthy) | σ ≈ err (healthy) | ✓ Yes | No filter divergence |
| Mean σ_pos (m) per time window | 0.33 → 0.42 → 0.35 → 0.25 → 0.27 | 0.40 avg | ✓ Converging | σ decreases late in T000001 |
| GPS sigma matches actual GPS error | σ=2.78 m, err=3.63 m avg | N/A (no GPS) | ⚠ No — fix R_gps | Sigma too low → over-trust |

# 6. Sensor Placement & Simulation Geometry Analysis

## 6.1  Were There Enough Sensors?

No. Here is the evidence from the data:

- GPS: Only 15 measurements over 13.78 seconds = ~1.1 Hz effective rate. For a vehicle traveling 246 m in 13.78 s (≈17.9 m/s average speed), a 1 Hz GPS gives a new fix every ~18 m of travel. That is far too sparse for a sub-meter tracking target. Standard vehicle GPS at 5–10 Hz would give a fix every 1.8–3.6 m.

- GPS coverage zone misses T000002 entirely — the antenna or zone boundary stops before segment 2.

- Camera: 52 frames over 13.78 s = ~3.8 Hz. With a 1.06 s max gap, the camera can miss a vehicle moving 18 m/s by as much as 19 m of travel between updates.

- DAS is well-placed (25 Hz) but provides only 1D along-fiber position — lateral information comes only from camera/GPS.

## 6.2  Segment Assignment Issue (T000001)

The tracker reports a seg1→seg2 transition for T000001 at t=7.39 s. The oracle ground truth shows the vehicle was in seg2 for the entire track duration. This means the estimated position was wrong enough during the early part of the track to be classified as seg1, even though the true position was in seg2. Two plausible explanations:

- The Kalman estimate drifts laterally far enough (Y-axis underestimated) to cross the segment boundary.

- The segment boundary centroid used for assignment does not match the oracle lane definition.

Recommended fix: assign segment from oracle position during simulation; use estimated position only in real-world deployment.

## 6.3  Camera Field-of-View Gap

T000001 receives camera measurements from t=0.11 s to t=12.65 s, but 37 of 52 measurements arrive before the segment transition. After the transition the camera rate drops sharply. This strongly suggests the camera's FOV does not fully cover the post-transition lane, or the vehicle exits the camera's angle range. The result: in the second half of the track the filter relies almost entirely on DAS, which cannot anchor lateral position.

# 7. Recommendations for Improving Simulation Quality

## 7.1  Fix GPS Noise Model (Priority: Critical)

The GPS measurement noise matrix R should reflect the true error distribution. Based on this simulation's data, the GPS sigma should be increased to approximately σ = 4–5 m (covering the observed range 1.0–8.1 m). This will reduce the Kalman gain for GPS updates and prevent the filter from pulling the estimate toward noisy GPS fixes with too much force.

Suggested parameter change: R_gps = diag([σ², σ²]) → increase σ from 2.78 to 4.5 m (matches observed 3.63 m mean + 1σ margin)

## 7.2  Increase GPS Coverage Rate and Zone (Priority: Critical)

- Raise GPS update rate from ~1 Hz to at least 5 Hz (preferably 10 Hz) for vehicles at 17 m/s.

- Extend GPS coverage zone to include segment 2 so T000002 receives corrections.

- If GPS antenna placement is fixed, add a second GPS receiver at the seg2 entry point.

## 7.3  Increase Camera Rate / Add Second Camera (Priority: High)

- Raise camera frame rate to 10 Hz (from ~4 Hz) to close the 1-second gap windows.

- Add a second camera with FOV covering the post-transition zone. Camera is the most accurate sensor — more camera = better tracking.

- Alternatively: reduce the camera max gap from 1.06 s to <0.3 s by widening FOV or adding overlap.

## 7.4  Tune Kalman Process Noise Q — Lateral Dynamics (Priority: High)

All peak errors show Y-axis (lateral) lag. The current Q matrix is likely isotropic or set for straight-line motion. Consider:

- Increase Q_yy (lateral position noise) to allow the filter to track lateral maneuvers without fighting the motion model.

- If the vehicle model is constant acceleration, test a constant velocity model — fewer parameters to misspecify.

- Alternatively, add a process noise adaption step that increases Q after N consecutive prediction-only steps.

## 7.5  DAS — Tighten End-of-Track Gate (Priority: Low)

- 191 DAS measurements are discarded because the vehicle had already left the zone. The DAS emitter could use the track lifetime to stop generating measurements earlier, reducing wasted filtering steps and potential ghost-detection risk.

## 7.6  Segment Assignment (Priority: Medium)

- For simulation validation: use oracle position to assign segment IDs, not the Kalman estimate.

- For production: add a hysteresis buffer at segment boundaries so a single noisy estimate cannot trigger a transition.

- Check segment centroid definitions against oracle lane boundaries — the mapping may have an offset.